CANADIAN METEOROLOGICAL

MARKUP LANGUAGE

SPECIFICATION

Version 3.01

History of the document

| Version no | Date | Comments | Reviewers |
| --- | --- | --- | --- |
| 3.01 | 2007-01-30 | Changed wind-speed to be optional, and the value of wind-speed and gust-speed to be nonNegativeInteger. | Jianling Hua |
| 3.0 | 2007-01-11 | Added scribe-cloud-cover-ceiling-codeType for attribute ‘ceiling-code’ of element cloud-cover. | Jianling Hua |
| 3.0 | 2006-12-06 | Added the meteocode-forecast node under forecast, added pavement-forecast node to hold the elements under original forecast node. Removed the deplicated inclusion of the xsd files. | Jianling Hua |
| 2.1 | 2006-08-23 | Removed sensors from metadata and moved the information to an instrument level. | Todd Billings |
| 2.02 | 2005-12-13 | Changed height element name of the subsurface sensor node to sensor-depth. | Todd Billings |
| 2.01 | 2005-11-30 | Updated elevation-units types table, air-quality types table. Added timezone types table. Updated timezoneType complex type structure definition. Added timezonetypeType simple type structure definition. Added stationStateType and instrumentStateType stationStateQualifierType structure definitions. Updated CMML data elements section with new element definitions ( station-state and instrument-state) | Todd Billings |
| 2.0 | 2005-08-24 | Restructured for inclusion of metadata, observation-series, and forecasting nodes. | Todd Billings |
| 1.0 | 2005-06-29 | Modified and restructured to finalize version 1.0 | Yungjae Cho |
| 1.0 | 2005-05-12 | Inserted “network-station-id” and multiple sources | Yungjae Cho Evan Leclair |
| 0.9 | 2005-04-27 | Restructuration and finalisation of document for version 1.0. | François Gauvin Gilles Larose Evan Leclair |
| 0.1 | 2004-06-15 | Creation of document. | François Gauvin |

For more information, refer to the CMML_control_document

Table of contents

1.	Overview	1

1.1.	CMML Control Document	1

1.2.	CMML Requirements	1

1.3.	Meteocode-Forecast Requirements:	1

1.4.	CMML Structure and Type Definitions	1

1.5.	CMML Schema Organization	1

1.6.	CMML Samples	1

2.	CMML Data Element Nomenclature	2

3.	CMML Framework Elements	3

3.1.	<cmml>	3

4.	CMML Header Elements	4

4.1.	<product>	4

4.2.	<source>	5

5.	CMML Data Elements	6

5.1.	<forecast>	6

5.1.1.	<meteocode-forecast>	6

5.1.1.1.	<location>	6

5.1.1.2.	<parameters>	6

5.1.1.2.1.	<accum-list>	6

5.1.1.2.2.	<cloud-list>	7

5.1.1.2.3.	<freezing-spray-list>	8

5.1.1.2.4.	<precipitation-list>	9

5.1.1.2.5.	<ice-cover-list>	10

5.1.1.2.6.	<probability-of-precipitation-list>	10

5.1.1.2.7.	<snow-level-list>	11

5.1.1.2.8.	<temperature-list>	11

5.1.1.2.9.	<UV-index-list>	12

5.1.1.2.10.	<visibility-list>	12

5.1.1.2.11.	<warning-list>	13

5.1.1.2.12.	<wave-height-list>	14

5.1.1.2.13.	<wind-list>	14

5.1.2.	<pavement-forecast>	15

5.1.2.1.	<location>	15

5.1.2.2.	<time-layout>	17

5.1.2.3.	<parameters>	18

5.1.2.3.2.	<pressure>	18

5.1.2.3.3.	<wind>	18

5.1.2.3.4.	<temperature>	19

5.1.2.3.5.	<precipitation>	20

5.1.2.3.6.	<radiation>	21

5.1.2.3.7.	<visibility>	22

5.1.2.3.8.	<pavement>	22

5.1.2.3.9.	<subsurface>	23

5.1.2.3.10.	<air-quality>	24

5.1.2.3.11.	<physical-status>	25

5.1.2.3.12.	<humidity>	26

5.1.2.3.13.	<snow>	27

5.1.2.3.14.	<weather>	28

5.1.2.3.15.	<extension>	28

5.2.	<observation-series>	29

5.2.1.	<origin>	29

5.2.2.	<location>	29

5.2.3.	<observation>	30

5.2.3.2.	<pressure>	30

5.2.3.3.	<wind>	31

5.2.3.4.	<temperature>	31

5.2.3.5.	<precipitation>	31

5.2.3.6.	<radiation>	32

5.2.3.7.	<visibility>	32

5.2.3.8.	<pavement>	32

5.2.3.9.	<subsurface>	33

5.2.3.10.	<air-quality>	33

5.2.3.11.	<physical-status>	33

5.2.3.12.	<humidity>	33

5.2.3.13.	<weather>	34

5.2.3.14.	<snow>	34

5.2.3.15.	<snapshot-camera>	34

5.2.3.16.	<extension>	35

5.3.	<metadata>	35

5.3.1.	<location>	35

5.3.1.3.	<region-metadata>	35

5.3.1.4.	<station-metadata>	36

5.3.1.4.24.	<pressure-instrument>	39

5.3.1.4.25.	<wind-instrument>	42

5.3.1.4.26.	<temperature-instrument>	46

5.3.1.4.27.	<precipitation-instrument>	49

5.3.1.4.28.	<radiation-instrument>	52

5.3.1.4.29.	<visibility-instrument>	56

5.3.1.4.30.	<pavement-instrument>	59

5.3.1.4.31.	<subsurface-instrument>	63

5.3.1.4.32.	<air-quality-instrument>	66

5.3.1.4.33.	<humidity-instrument>	69

5.3.1.4.34.	<snow-instrument>	73

5.3.1.4.35.	<weather-instrument>	76

5.3.1.4.36.	<snapshot-camera-instrument>	80

5.3.1.4.37.	<extension-instrument>	83

5.3.2.	<supplementary-metadata>	86

5.3.2.2.	<categorical-definitions>	87

5.3.2.3.	<conversion-definitions>	87

5.3.2.4.	<sampling-definitions>	87

6.	Common Element and Attribute Definitions	89

6.1.	<point>	89

6.4.	<qualifier>	89

6.5.	<value>	89

6.6.	<qc>	89

6.7.	<contact>	90

6.8.	<equipment-information>	90

6.9.	<exception>	91

6.10.	<excep-category>	91

7.	XML Considerations	92

7.1.	Namespace:	92

7.2.	Schema:	92

7.3.	Character Set:	92

Appendix A	Requirements	A-1

Appendix B	Meteocode-Forecast Requirements	B-1

Appendix C	Structure and Type Definitions	C-1

Appendix D	Valid Head Types	D-1

Appendix E	Valid Forecast Types	E-1

Appendix F	Valid Observation Types	F-1

Appendix G	Valid Metadata Types	G-1

Appendix H	Schema Organization	H-1

Appendix I	References	I-1

Glossary	I-2

Quick Reference	I-3

- Overview : This document defines the Canadian Meteorological Markup Language (CMML). CMML is a new XML language which is being developed to initially support the exchange of road weather data between all Provinces and Territories of Canada (PTC) and Environment Canada (EC). However, the specification is being written with enough flexibility to accommodate exchange of other environmental science and weather applications. In order to support the interoperability, the future versions of CMML, all non core data elements, e.g. site specific data elements will be handled by using extension tags.

  - CMML Control Document : The control document “CMML Control Document” defines the copyright and document distribution information relating to these specifications in addition to making the follow-up of its evolutions.

  - CMML Requirements: See Appendix A

  - Meteocode-Forecast Requirements: See Appendix B

  - CMML Structure and Type Definitions: See Appendix C

  - CMML Schema Organization: Appendix D

  - CMML Samples: In addition to providing a definition of the elements and their attributes, CMML sample files can be found separate of this document. Please refer to the control document to know more.

- CMML Data Element Nomenclature: This specification uses the following approach to describing elements:

  - Element and attribute names are all lower case.

  - Element and attribute names use a hyphen (“-“) to separate multiple word names so as to improve readability (ex. <creation-date>). Attribute and element names avoid the use of abbreviations to enhance readability.

  - Within this document, child elements are nested in a sub-paragraph under their parents.

  - Attributes are also nested but do not have the angle brackets (“<>”) and are italicized.

  - The element’s and attribute’s type is provided in braces (“{}”). For more information on each type refer to Appendix C .

  - If the element can occur zero or more times, an asterisk (“*”) is placed after its name. See section 5.2.3.2 for an example.

  - If the element occurs zero or one times, a question mark (“?”) follows its name. See section 5.3.2 for an example.

  - If the element must appear at least once, a plus sign (“+”) trails its name. See section 5.2.3 for an example.

  - Element names without a special trailing character must occur exactly once. Section 5.2.1 provides an example of an element that is required exactly once.

  - The order of element descriptions in the paragraphs below is not significant. Any required ordering of elements is specified in Appendix C .

- CMML Framework Elements:

  - <cmml> {cm:cmmlType}: The root element for CMML.

    - version {cm:non-emptyString}: Indicates which version of CMML the instance contains.

    - <head> {cm:headType}: Contains the header for the CMML instance. See section 4 for elements found in the <head> element.

    - <data> {cm:dataType}: Contains the environmental data. See section 5 for child elements of the <data> element.

- CMML Header Elements: CMML header provides information about the CMML product and the data it contains. These elements are children of the <head> element.

  - <product>   {cm:productType}: Holds header information about the product.

    - operational-mode {cm:operational-modeType}: Defines the status of the product. Applications can review the content of this element to determine if they should perform further processing. Sample values include “test”, “developmental”, “experimental”, and “official” product.

      - Test Product: Indicates that this is an instance of an existing CMML product that contains some change being evaluated by a CMML development team. Users will typically not process this product.

      - Developmental Product: A new product that is not yet ready for public evaluation or use.

      - Experimental Product: Product is available for testing and evaluation for a specified, limited time period for the explicit purpose of obtaining customer feedback.

      - Official Product: Identifies an instance of an established CMML product.

    - <title> {xsd:string} [?]: Provides a concise summarization of what this CMML product contains.

    - <field> {cm:fieldType}: Specifies the general area within the environmental sciences that the data contained in the CMML instance is from. Example values include “meteorological” “hydrological” “oceanographical” “land surface” and “space”

    - <category> {cm:categoryType}: Defines the specific category that the product belongs to. Example values include “observation”, “forecast”, “analysis”, “statistics” and “metadata”. If a category is defined as “metadata”, only metadata blocks should be included in the cmml file, otherwise metadata must not be included.

    - <creation-date> {cm:creation-dateType}: The date and time that the product was prepared.

      - refresh-frequency {xsd:duration}: Used by the production center to help users know how often to return for updated data. The suggested refresh frequency will help well mannered users know what the provider believes is a reasonable time between repeated accesses of the system.

    - <current-issue> {xsd:dateTime}[?]:  The issue date and time of this product.

    - <next-issue> {xsd:dateTime} [?]:  The date and time of the next issue of this product.

    - <timezone>1 {xsd:string}[?]:  Provides the timezone of the forecast.

    - <status> {cm:scribe-statusType}[?]:  Status of the bulletin, used for marine forecasts.  See Table D-4:  Valid types for product status.

    - <type>1 {cm:scribe-typeType}[?]:  See Table D-5: Valid types for product type.

    - <initials>1 {xsd:string}[?]:  Identification of product authors.

    - <valid-begin-time>1 {xsd:dateTime}[?]:  Date and time at which the validity of the forecast begins.

    - <valid-end-time>1 {xsd:dateTime}[?]:  Date and time at which the validity of the forecast ends.

    - <amended-regions> {cm:scribe-amended-regionsType}[?]:  List of amended or corrected regions in the forecast.

      - <location> {cm:scribe-locationType} [+]:  See section 5.1.1.1.

  - <source>   {cm:sourceType}[+] : Holds information about the product’s source and links to credit and disclaimer information.

    - <more-information> {xsd:anyURI}[?] : A link to the web page of the forecast’s source or a more complete forecast.

    - <production-center> {cm:production-centerType}: Production Center identifies which organization creates the product.

      - <sub-center> {xsd:string} [?]: The part of the production center that prepared the product.

    - <disclaimer> {xsd:anyURI} [?]: The URL containing a disclaimer regarding the data.

    - <credit> {xsd:anyURI} [?]: The URL used to credit the source of the data.

    - <credit-logo> {xsd:anyURI} [?]: The image link used with the credit URL to acknowledge the data source.

    - <feedback> {xsd:anyURI} [?]: A URL to a web page used to provide the production center comments on the product.

- CMML Data Elements: These elements hold the environmental data. They are children of the <data> element. At least one child element (<forecast>, <observation-series>, or <metadata>) must be present in a <data> element.

  - <forecast>   {cm:forecastType}[*]: Defines forecast information. At least one child element (<meteocode-forecast> or <pavement-forecast>) must be present in a <forecast> element.

    - <meteocode-forecast> {cm:scribe-forecastType}[*]: Weather elements forecasted by Environment Canada through the Scribe application forecast product.

      - <location> {cm:scribe-locationType}[+]: Defines the location for the data contained in the element <meteocode-forecast>. The element must contain one of the following elements: <point> or <msc-zone-code>.

        - <point> {cm:scribe-pointType}: Element used to define the geographical point for which the data is valid.

          - <latitude> {xsd:decimal}: The latitude of the point where the data is valid.

          - <longitude> {xsd:decimal}: The longitude of the point where the data is valid

        - <msc-zone-code> {cm:scribe-msc-zone-codeType}: Contains the Meteorological Service of Canada forecast zone code for which the data is valid.

          - status {cm:scribe-msc-zone-code-statusType}[?]: Indicates the status for the region, used for marine forecasts.  See Table E-33: Valid values for meteocode-forecast msc zone code status.

        - <msc-zone-name> {cm:scribe-msc-zone-nameType}[+]:  Name of the region.

          - lang {cm:scribe-langType}: Language. Values are “en” for English or “fr” for French.

      - <parameters> {cm:scribe-parametersType}[+]: Holds the environmental data. At least one child element must be present in a <parameters> element.

        - <accum-list>  {cm:scribe-accum-listType}[*]: Container for descriptions of accumulation of precipitation.

          - units : Defines the units of the accumulation. The default value is “mm”. See Table E-49:  Valid unit types for meteocode-forecast accumulation.

          - <accum-amount>  {cm:scribe-accum-amountType} [*]

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

            - type { cm:scribe-accum-typeType }: Identifies the type of accumulation of precipitation.   See Table E-26: Valid values for meteocode-forecast accumulation types.

            - <lower-limit> {cm:scribe-accum-lower-limitType}: Lower limit value of precipitation amount.

              - cumul-inf {xsd:decimal}: Value of lower cumulative precipitation amount of current precipitation amount type.

            - <upper-limit> {cm:scribe-accum-upper-limitType}: Upper limit value of precipitation amount.

              - cumul-sup {xsd:decimal}: Value of upper cumulative precipitation amount of current precipitation amount type.

        - <cloud-list> {cm:scribe-cloud-listType}[*]: Container for cloud cover descriptions

          - units {cm:scribe-cloud-unitsType}: Defines the units of the cloud cover. The default value is in tenth of the sky “deci”. See Table E-50:  Valid unit types for meteocode-forecast cloud.

          - <cloud-cover> {cm:scribe-basic-cloud-coverType} [*]: Value of the cloud cover.

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid

            - ceiling-code {cm:scribe-cloud-cover-ceiling-codeType}[?]:  Specifies the height above ground of the main cloud cover. These codes are produced automatically by Scribe and are generally not modified by the forecaster. If not modified, the ceiling value will not be present if cloud cover is "0". Although the definition of a ceiling (aviation rules) requires 6/10 of cloud cover, ceiling values will be produced at all cloud cover greater than "0" and will represent the height of the most representative cloud layer. See Table E-56:  Valid values for meteocode-forecast cloud cover ceiling-code

          - <exception> {cm:scribe-cloud-cover-exceptionType} [*]: See section 6.9 for a description of an exception.

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

            - <excep-category> {cm:scribe-excep-categoryType}:  See section 6.9 for definition.

            - <cloud-cover>”  {cm:scribe-cloud-coverType}: A different value of cloud cover for the exception.

              - start {xsd:dateTime}: Specifies the start time of the period of time for which the data of the exception is valid.

              - end {xsd:dateTime}: Specifies the end time of the period of time for which the data of the exception is valid.

              - ceiling-code {cm:scribe-cloud-cover-ceiling-codeType}[?]: See section 5.1.1.2.2.2.3

              - cover-start {xsd:nonNegativeInteger}: Value of cloud cover at exception start time.

              - cover-end {xsd:nonNegativeInteger}: Value of cloud cover at exception end time.

        - <freezing-spray-list> {cm:scribe-freezing-spray-listType}[*]: Container for the freezing spray, marine forecasts only.

          - <freezing-spray> {cm:scribe-freezing-sprayType}[*]

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

            - frequency {cm:scribe-freezing-spray-frequencyType}: See Table E-29: Valid types for meteocode-forecast freezing spray frequency.

            - intensity {cm:scribe-freezing-spray-intensityType}: See Table E-30: Valid types for meteocode-forecast freezing spray intensity.

            - occurrence{cm:scribe-freezing-spray-occurType}: See Table E-31: Valid types for meteocode-forecast freezing spray occurrence.

            - ice-cover-modifier {cm:scribe-freezing-spray-ice-coverType} [?]:   Ice cover modifiers applied to the freezing spray . See Table E-32: Valid types for meteocode-forecast freezing spray ice cover modifier.

            - <exception> {cm:scribe-freezing-spray-exceptionType} [?]: See section 6.9 for a description of an exception.

              - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

              - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

              - <excep-category> {cm:scribe-excep-categoryType}:  See section 6.9 for definition.

              - <freezing-spray> {cm:scribe-basic-freezing-sprayType} [*]: contains the same attributes/elements defined in section 5.1.1.2.3.1, but without an <exception>.

        - <precipitation-list> {cm:scribe-precipitation-listType}[*]:  Container for the precipitation values.

          - <precipitation-event> {cm:scribe-precipitation-eventType} [*]

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

            - type {cm:scribe-precipitation-typeType}: See Table E-37: Valid types for meteocode-forecast precipitation types.

            - frequency{cm:scribe-precipitation-frequencyType}:  See Table E-34: Valid types for meteocode-forecast precipitation frequency.

            - intensity{cm:scribe-precipitation-intensityType}:  See Table E-35: Valid types for meteocode-forecast precipitation intensity.

            - occurrence{cm:scribe-precipitation-occurType}:  See Table E-36: Valid types for meteocode-forecast precipitation occurrence.

            - <exception> {cm:scribe-precipitation-exceptionType} [?]:  See section 6.9 for a description of an exception.

              - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

              - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

              - <excep-category> {cm:scribe-excep-categoryType}: See section 6.9 for definition.

              - <precipitation-event> {cm:scribe-basic-precipitation-eventType}: Contains the same attributes/elements defined in section 5.1.1.2.4.1, but without  an <exception>.

            - <precipitation-event> {cm:scribe-precipitation-eventType} [*]: See section 5.1.1.2.4.1

              - <precipitation-event> {cm:scribe-precipitation-eventType} [*]: See section 5.1.1.2.4.1

- Note: Since precipitation events can occur simultaneously, three levels of precipitation types are possible. Description of each of these levels is available in this XML format. Level 2 will be imbedded in level 1 and level 3 will be imbedded in level 2.

        - <ice-cover-list>  {cm:scribe-ice-cover-listType}[*]: Container for the ice cover, marine forecast only.

          - units {xsd:string}: Defines the unit for the ice cover. The value is fixed at “%”.

          - <ice-cover> {cm:scribe-ice-coverType} [*]

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

        - <probability-of-precipitation-list> {cm:scribe-probability-of-precipitation-listType}[*]: Container for the probabilities of precipitation.

          - units:{xsd:string}: Defines the unit for the probability of precipitation. The value is fixed at “%”.

          - <probability-of-precipitation> {cm:scribe-probability-of-precipitation-Type}[*]

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

        - <snow-level-list> {cm:scribe-snow-level-listType}[*]: Container for snow level descriptions.

          - units {cm:scribe-snow-level-unitsType}: Defines the units of the snow level.  The default value is “m”. See Table E-51:  Valid unit types for meteocode-forecast snow level.

          - <snow-level> {cm:scribe-snow-levelType} [*]

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

        - <temperature-list> {cm:scribe-temperature-listType}[*]: Container for temperature descriptions.

          - type {cm:scribe-temperature-typeType}: Type of temperatures. See Table E-38: Valid values for meteocode-forecast temperature type.

          - units {cm:scribe-temperature-unitsType}: Defines the units of the temperature.  The default value is “celsius”. See Table E-52:  Valid unit types for meteocode-forecast temperature.

          - <temperature-value> {cm:scribe-temperature-valueType}[*]

            - trend {cm:scribe-temperature-trendType}[?]: Indicates a tendency. See Table E-40: Valid types for meteocode-forecast temperature trend.

            - ground-frost {xsd:string}[?]:  Indicate a ground frost status. See Table E-39:  Valid values for meteocode-forecast temperature ground-frost

            - start{xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime} : Specifies the end time of the period of time for which the data is valid.

            - <limit> {xsd:decimal}[?]: Climatological or sea-surface value of temperature.  Real number.  Value of -9999.00 indicates a missing value.

            - <lower-limit> {xsd:decimal}[?]:  Lower value of a range of temperature (air-temperature or dew-point).  Real number. Value of -9999.00 indicates a missing value.

            - <upper-limit> {xsd:decimal}[?]:  Upper value of a range of temperature (air-temperature or dew-point).  Real number.  Value of -9999.00 indicates a missing value.

          - <exception> {cm:scribe-temperature-exceptionType}[*]:  See section 6.9 for a description of an exception.

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

            - <excep-category> {cm:scribe-excep-categoryType}:  See section 6.9 for definition.

            - <temperature-value> {cm:scribe-temperature-valueType}:  See section 5.1.1.2.8.3 for definition.

        - <UV-index-list> {cm:scribe-UV-index-listType}[*]: Container for UV index forecast.

          - <UV-index> {cm:scribe-UV-indexType}[*]: Value of the UV index.  Real number that varies from 0 to 15.

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}  : Specifies the end time of the period of time for which the data is valid.

        - <visibility-list>  {cm:scribe-visibility-listType}[*]: Container for the visibility values.

          - <visibility> {cm:scribe-visibilityType} [*]

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime: Specifies the end time of the period of time for which the data is valid.

            - type {cm:scribe-visibility-typeType}: See Table E-42: Valid types for meteocode-forecast visibility.

            - frequency {cm:scribe-visibility-frequencyType}: See Table E-43: Valid types for meteocode-forecast visibility frequency.

            - <value>  {cm:scribe-valueType}[?]:  Distance of horizontal visibility

              - unit {cm:scribe-value-unitsType}:  Defines the units of visibility value.  The default value is “NM”. See Table E-53:  Valid unit types for meteocode-forecast visibility value.

            - <exception> {cm:scribe-visibility-exceptionType} [?]:  See 6.9 for a description of an exception.

              - start{xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

              - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

              - <excep-category>{cm:scribe-excep-categoryType}:  See section 6.9 for definition.

              - <visibility>{cm:scribe-basic-visibilityType}:  Contains the same attributes/elements defined in section 5.1.1.2.10.1 but without an <exception>.

        - <warning-list>{cm:scribe-warning-listType}[*]: Container for warning descriptors.  Three levels of warning can co-exist at the same time.

          - <warning-event> {cm:scribe-warning-eventType}[*]:  Description of a warning event.

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

            - type {cm:scribe-warning-typeType}: Warning category.  SeeTable E-44: Valid types for meteocode-forecast warning type

            - code {cm:scribe-warning-codeType}: See Table E-45: Valid values for meteocode-forecast warning codes.

            - status {cm:scribe-warning-statusType}: See Table E-46: Valid values for meteocode-forecast warning status.

            - <warning-event>{cm:scribe-warning-eventType}[*]:  Second level of warning.

              - <warning-event>{cm:scribe-warning-eventType}[*]:  Third level of warning.

        - <wave-height-list> {cm:scribe-wave-height-listType}[*]: Container for the wave height values.

          - units {cm:scribe-wave-height-unitsType}: Defines the units of the wave height. The default value is “m”. See Table E-54:  Valid unit types for meteocode-forecast wave height.

          - <wave-height>{cm:scribe-wave-heightType}[*]: Container for the wave height.

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

            - ice-cover-modifier  {cm:scribe-wave-height-ice-coverType}[?]:   Ice cover modifiers applied to wave height. See Table E-41: Valid types for meteocode-forecast wave-height ice cover modifier.

            - <lower-limit>{xsd:decimal}:  Lower value of a range of height.  Real number.

            - <upper-limit>{xsd:decimal}:  Upper value of a range of height. Real number.

        - <wind-list> {cm:scribe-wind-listType}[*]: Container for wind event descriptions.

          - units {cm: scribe-wind-unitsType}: Defines the units of the wind/gust speed.  The default value is “kmh”. See Table E-55:  Valid unit types for meteocode-forecast wind.

          - <wind> {cm:scribe-windType}[*]:  Container for a wind event.

            - start {xsd:dateTime}: Specifies the start time of the period of time for which the data is valid.

            - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

            - direction {cm:scribe-wind-directionType}: Wind direction.  See Table E-47: Valid values for meteocode-forecast wind direction.

            - modificator {cm:scribe-wind-modificatorType} [?]: Modifiers applied to wind  See Table E-48: Valid values for meteocode-forecast wind modificators.

            - <wind-speed> {cm:scribe-wind-speedType}[?]: Wind speed.  The default unit value is in “kmh”

              - <lower-limit> {xsd:nonNegativeInteger}: Lower value of the speed.  Non negative integer.

              - <upper-limit> {xsd: nonNegativeInteger }: Upper value of the speed.  Non negative integer.

            - <gust-speed> {cm:scribe-gust-speedType}[?]:  Gust speed.

              - <lower-limit> {xsd: nonNegativeInteger }: Lower value of the speed.  Non negative integer.

              - <upper-limit> {xsd: nonNegativeInteger }: Upper value of the speed.  Non negative integer.

            - <exception> {cm:scribe-wind-exceptionType}[?]: See section 6.9 for a description of an exception.

              - start : {xsd:dateTime}Specifies the start time of the period of time for which the data is valid.

              - end {xsd:dateTime}: Specifies the end time of the period of time for which the data is valid.

              - <excep-category> {cm:scribe-excep-categoryType}: See section 6.9 for definition.

              - <wind> {cm:scribe-basic-wind-type}: Contains the same attributes/elements defined in section 5.1.1.2.13.2 but without an <exception>.

    - <pavement-forecast> {cm:pavement-forecastType}[*]: Defines pavement forecast information.

      - <location>  {cm:locationType} [+]: Defines the location that provides the data contained in the element <forecast>. The element must contain exactly one of the following elements; <point>, <city>, <nws-zone>, <area>, <route>, or <station>.

        - <location-key> {xsd:string} : The location-key element is used to relate the location to its corresponding parameters. Must be unique.

        - <point> see section 6.1.

        - <city> {cm:cityType} [?]: Contains the city name for which the data is.

          - province {cm:provinceType}: The two digit abbreviation for the province that the city resides.

          - summarization: see section 6.2.

        - <nws-zone> {xsd: string} [?]: Contains the National Weather Service forecast zone code for which the data is valid.

          - province {cm:provinceType}: Defines the two letter province ID.

          - summarization: see section 6.2

        - <area>  {cm:areaType} [?]: A geometrical shape may be used to define which grid points the data represents. The element must contain exactly one of its child elements.

          - <circle> {cm:circleType} [?]: A circular area about a grid point. The area can contain any number of grid points which are summarized.

            - <point> see section 6.1.

            - <radius> {cm:radiusType}: The distance from the center point of the circle to edge of the circular area.

              - radius-units {cm:radius-unitsType): The units of the radius measurement. Example values include “statute-miles” and “kilometers”.

          - <rectangle> {cm:rectangleType} [?]: A rectangular area is defined by two latitude and longitude pairs. These pairs form a diagonal from which a rectangle can be consistently constructed. The area can contain any number of grid points which are summarized.

            - <point> see section 6.1.

            - <point> see section 6.1.

          - <polygon> {cm:polygonType} [?]: A polygon area which is defined by three or more latitude and longitude pairs. The area can contain any number of grid points which are summarized.

            - <point> see section 6.1.

            - <point> see section 6.1.

            - <point> see section 6.1.

        - <route> {cm:linestringType} [?] : A linear road network which is defined by two or more latitude and longitude pairs.

          - <point> see section 6.1.

          - <point> see section 6.1.

        - <station> {cm: originType} [?]: Defines the station that provides the data.

          - type {cm:origintypeType} : Defines the type of station. Example values are “station” and “region”. See Table E-11: Valid pavement-forecast origintype types

          - <id> {cm: identifierType} [+]: Contains an id of station.

            - type {cm:identifiertypeType} : Defines the type of ID. Example values are “client” and “network”. See Table E-10: Valid pavement-forecast identifiertype types.

      - <time-layout>  {cm:time-layoutElementType} [+]: Contains the start and stop valid times and any associated period names for the data. Since different environmental parameters have different time schemes (valid at different interval and available for different lengths of time into the future), there will be one <time-layout> element for each of these unique temporal configurations. Each data parameter will reference exactly one of these time layouts.

        - time-coordinate {cm:time-coordinateType}: The time coordinate can be either “local” or “UTC”. See Table E-21: Valid pavement-forecast time-coordinate types.

        - summarization see section 6.2.

        - <layout-key> {cm:layout-keyType}: The key used to associate this time layout with a particular parameter element.

        - <start-valid-time> {cm:start-valid-timeType} [+]: The start time of the period of time for which the data is valid.

          - period-name {xsd:string} [?]: Contains the name associated with this time interval (ex. TODAY).

        - <end-valid-time> {xsd:dateTime} [*]: The end time of the period of time for which the data is valid. The absence of this element indicates that the element is valid at a specific time.

      - <parameters> {cm:parametersType} [+]: Holds the environmental data.

        - applicable-location {xsd:string} : The applicable-location attribute is used to relate the location to a particular list of parameters.

        - <pressure>  {cm:pressureType} [*]: Container for the pressure values.

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of pressure. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for pressure. Example values include “atmospheric” and “average-atmospheric”. See Table E-15: Valid types for pavement-forecast pressure

          - height {xsd:nonNegativeInteger}[?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the pressure value. The default value is “mb”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value.

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data.

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The pressure value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name>  {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <wind> {cm:windType} [*]: Container for the wind values.

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of wind. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for wind. Example values include “average-direction” and “speed”. See Table E-24: Valid types for pavement-forecast wind.

          - height {xsd:nonNegativeInteger}[?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the wind values. Example values include “deg” and “km/h”. The default value is “deg”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value.

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data.

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The wind value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <temperature> {cm:temperatureType} [*]: Container for temperature data.

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of temperature. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for temperature. Example values include “maximum-air-temperature”, “minimum-air-temperature”, “air-temperature”, “dew point”, “wet bulb”. See Table E-20: Valid types for pavement-forecast temperature.

          - height {xsd:nonNegativeInteger}[?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the temperature value. Example values include “degF”, “degC”, and “K”. The default value is “degC”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value.

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data.

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The temperature value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <precipitation> {cm:precipitationType} [*]: Container for the precipitation values

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of precipitation. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for precipitation. Example values include “rate”, “situation”. See Table E-14: Valid types for pavement-forecast precipitation.

          - height {xsd:nonNegativeInteger}[?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the precipitation value. Example values include “inches” and “millimeters”. The default value is “mm”. See Table E-22: Valid pavement-forecast unit types

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value.

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The precipitation type parameter’s value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <radiation> {cm:radiationType} [?]: Container for the radiation values .

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of radiation. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for radiation. Example values include “total-sun”, “total-radiation”, “average-short-wave-up” and “average-short-wave-down. See Table E-16: Valid types for pavement-forecast radiation

          - units {cm:unitType}[?]: Defines the units of the radiation values. Example values include “W/m²”. The default value is “W/m²”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - height {xsd:nonNegativeInteger}[?]: Defines the height where values was measured. The height unit is centimeters.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The radiation value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <visibility> {cm:visibilityType} [*]: Container for the visibility values .

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of visibility. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for visibility. Example values include “distance” and “situation”. See Table E-23: Valid types for pavement-forecast visibility.

          - height {xsd:nonNegativeInteger}[?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the visibility value. Example values include “km” and “m”. The default value is “m”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The visibility value Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <pavement> {cm:pavementType} [*]: Container for the pavement values .

          - lane-number { xsd:nonNegativeInteger } [?]: Specifies the lane number where the values are measured. The default value is 1.

          - index2 { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of pavement. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for pavement. Example values include “surface-status”, “temperature” and “salinity”. See Table E-12: Valid types for pavement-forecast pavement.

          - height {xsd: integer}[?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the pavement value. Example values include “degC”, “degF” and “K”. The default value is “degC”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The pavement value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <subsurface> {cm:subsurfaceType} [*]: Container for the subsurface values.

          - lane-number { xsd:nonNegativeInteger } [?]: Specifies the lane number where the values are measured. The default value is 1.

          - index3 { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of subsurface. The default value is 1.

          - type {xsd:string} : Specifies the type of variable measured for subsurface. Example values include “temperature”, “moisture” and “average-temperature”. See Table E-18: Valid types for pavement-forecast subsurface.

          - sensor-depth {xsd:nonNegativeInteger}: Defines the depth where the measure was taken. The depth will be measured in centimeters.

          - units {cm:unitType}[?]: Defines the units of the subsurface value. Example values include “degC”, “degF” and “K”. The default value is “degC”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The subsurface value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <air-quality> {cm:airQualityType} [*]: Container for the air quality values .

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of air-quality. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for air quality. Example values include “carbon-monoxide”, “carbon-dioxide” and “ozone”. See Table E-6: Valid types for pavement-forecast air-quality.

          - height {xsd:nonNegativeInteger}[?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the air quality value. Example values include “ppm” and “ppb”. The default value is “ppb”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The air quality value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <physical-status> {cm:physicalStatusType} [*]: Container for the physical status values .

          - type {xsd:string}: Specifies the type of variable measured for physical status. Example values include “door-open” and “battery-minimum-voltage”. See Table E-13: Valid types for pavement-forecast physical-status.

          - units {cm:unitType}[?]: Defines the units of the physical status value. Example values include “V” and “bitmap”. The default value is “V”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - <value> {xsd:anyType} [+]: The physical status value. Missing values are represented by an empty element and xsi:nil=”true” .

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

        - <humidity> {cm:humidityType} [?]: Container for humidity values .

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of humidity. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for humidity. Examples values include “relative-humidity” and “average-relative-humidity”. See Table E-9: Valid types for pavement-forecast humidity.

          - height {xsd:nonNegativeInteger} [?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the humidity values. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The humidity value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <snow> {cm:snowType} [?]: Container for snow values .

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of snow. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for snow. See Table E-17: Valid types for pavement-forecast snow

          - height {xsd:nonNegativeInteger} [?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the snow values. Example values include “cm” and “mm”. The default value is “cm”.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The snow value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <weather> {cm:weatherType} [?]: Container for weather values .

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of weather. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for weather. See Table E-25: Valid types for pavement-forecast weather.

          - height {xsd:nonNegativeInteger} [?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the weather values. The default value is “%”. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The weather value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

        - <extension> {cm:extensionType} [*]: Container for any type of values.

          - index { xsd:nonNegativeInteger } [?]: Specifies the index for a same type of extension. The default value is 1.

          - type {xsd:string}: Specifies the type of variable measured for extension.

          - height {xsd:nonNegativeInteger} [?]: Defines the height where values was measured. The height unit is centimeters.

          - units {cm:unitType}[?]: Defines the units of the extension values. See Table E-22: Valid pavement-forecast unit types.

          - categorical-code {xsd:string}[?]: Defines the name of the code that represent the values. This code can be used to do the mapping with a conversion table that defines the meaning of each possible value that is already known by the requester.

          - time-layout {cm:time-layoutType}: see section 6.3.

          - categorical-table {cm:categorical-tableType} [?]: Foreign key to a list of categories that define the meaning of the value .

          - conversion-table {cm:conversion-tableType} [?]: Foreign key to a list of conversions tables that provide a equivalent value for the data .

          - sampling-table {cm:sampling-tableType} [?]: Foreign key to a list of sampling tables that provide precision for the data and how it was acquired.

          - <value> {xsd:anyType} [+]: The extension value. Missing values are represented by an empty element and xsi:nil=”true”.

          - <name> {xsd:string} [?]: The name of this parameter. The name value can be used for display purposes.

  - <observation-series>  {cm:observation-seriesType}[*]: Defines a series of observations for a location.

    - <origin> {cm:originType} : The originator of the observation.

      - type{cm:origintypeType}: What type of originator. Example values include “station” and “region”.

      - <id> {cm:identifierType}[+] : A key that uniquely identifies the originator

        - type {cm:identifiertypeType}: What type of identifier. Example values include “client” and “network”.

    - <location> {cm:locationType}[?] : The location where the observation is being taken. This is an optional element that is included when the location of an observation changes from one observation to the next. Examples include mobile stations, or ad-hoc regions.

      - <point> : see section 6.1..

      - <area> {cm:areaType} [?]: A geometrical shape may be used to define which grid points the data represents. The element must contain exactly one of its child elements.

        - <circle>  {cm:circleType} [?]: A circular area about a grid point. The area can contain any number of grid points which are summarized.

          - <point>: see section 6.1..

          - <radius> {cm:radiusType}: The distance from the center point of the circle to edge of the circular area.

          - radius-units {cm:radius-unitsType}: The units of the radius measurement. Example values include “statute-miles” and “kilometers”.

        - <rectangle> {cm:rectangleType} [?]: A rectangular area is defined by two latitude and longitude pairs. These pairs form a diagonal from which a rectangle can be consistently constructed. The area can contain any number of grid points which are summarized.

          - <point> see section 6.1.

          - <point> see section 6.1.

        - <polygon>{cm:polygonType} [?]: A polygon area which is defined by three or more latitude and longitude pairs. The area can contain any number of grid points which are summarized.

          - <point>  see section 6.1.

          - <point> see section 6.1.

          - <point> see section 6.1.

      - <route> {cm:linestringType} [?] : A linear road network which is defined by two or more latitude and longitude pairs.

        - <point> see section 6.1.

        - <point> see section 6.1.

    - <observation>  {cm:observationType}[+] : Holds the environmental data. At least one child element must exist.

      - valid-time {xsd:dateTime}: The date, time, and timezone of the observation.

      - <pressure>  {cm:pressureObsType}[*]: Container for pressure measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of pressure

        - type {cm:pressureTypeType}: The type of pressure measurement the pressure node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <wind>  {cm:windObsType}[*]: Container for wind measurements

          - index {xsd:nonNegativeInteger}: Specifies the index for a same type of wind

          - type{cm:windTypeType}: The type of wind measurement the wind node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <temperature>{cm:temperatureObsType}[*]: Container for temperature measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of temperature

        - type {cm:temperatureTypeType}: The type of temperature measurement the temperature node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <precipitation>{cm:precipitationObsType}[*]: Container for precipitation measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of precipitation

        - type {cm:precipitationTypeType}: The type of precipitation measurement the precipitation node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <radiation>{cm:radiationObsType}[*]: Container for radiation measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of radiation

        - type {cm:radiationTypeType}: The type of radiation measurement the radiation node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <visibility>{cm:visibilityObsType}[*]: Container for visibility measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of visibility

        - type {cm:visibilityTypeType}: The type of visibility measurement the visibility node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <pavement>{cm:pavementObsType}[*]: Container for pavement measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of pavement

        - type {cm:pavementTypeType}: The type of pavement measurement the pavement node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <subsurface>{cm:subsurfaceObsType}[*]: Container for subsurface measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of subsurface

        - type {cm:subsurfaceTypeType}: The type of subsurface measurement the subsurface node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <air-quality>  {cm:air-qualityObsType}[*]: Container for air-quality measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of air-quality

        - type {cm:air-qualityTypeType}: The type of air-quality measurement the air-quality node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <physical-status>{cm:physical-statusObsType}[*]: Container for physical-status measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of physical-status

        - type {cm:physical-statusTypeType}: The type of physical-status measurement the physical-status node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <humidity>{cm:humidityObsType}[*]: Container for humidity measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of humidity

        - type {cm:humidityTypeType}: The type of humidity measurement the humidity node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <weather> {cm:weatherObsType}[*]: Container for weather measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of weather

        - type {cm:weatherTypeType}: The type of weather measurement the weather node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <snow> {cm:snowObsType}[*]: Container for snow measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of snow

        - type {cm:snowTypeType}: The type of snow measurement the snow node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <snapshot-camera> {cm:snapshot-cameraObsType}[*]: Container for snapshot-camera measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of snapshot-camera

        - type {cm:snapshot-cameraTypeType}: The type of snapshot-camera measurement the snapshot-camera node is describing.

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

      - <extension> {cm:extensionObsType}[*]: Container for extension measurements

        - index {xsd:nonNegativeInteger}: Specifies the index for a same type of extension

        - <qualifier> see section 6.4

        - <value> see section 6.5

        - <qc> see section 6.6

  - <metadata>   {cm:metadataType}[*]: Contains metadata.

    - <location>   {cm:location-metadataType} [?] Metadata information about a specific location.

      - type  {cm:origintypeType} What type of location. Example values include “station” and “region”

      - <id> {cm:identifierType}[+] : A key that uniquely identifies the location

        - type {cm:identifiertypeType}: What type of identifier. Example values include “client” and “network”.

      - <region-metadata> {cm:region-metadataType} [?] Metadata information about a region.

        - <name>  {xsd:string} [?] A unique identifier for this location assigned by the client.

        - <description>  {xsd:string} [?] A text description of this location that includes any relevant details that cannot be further specified in the CMML metadata.

        - <city>  {xsd:string} [?] The city that contains or administers this location. Omit if not applicable.

        - <country> {cm:ISO-country-codeType} [?] Species the two-letter ISO country code for the country that contains this location.

        - <province>  {cm:CA-province-codeType} [?] If the location is within Canada, this specifies the two-letter identifier code for the associated province or territory.

        - <state>  {cm:US-state-codeType} [?] If the location is within the USA, this specifies the two-letter identifier code for the associated state or possession.

        - <climatic-zone>  {xsd:string} [?] Specifies the climatic zone of this location, if available.

        - <district>  {xsd:string} [?] Specifies the district that contains this location for those clients that group locations in that manner.

        - <timezone>  {cm:timezoneType} [?] Specifies the time zone and time zone offset at this location.

        - <data-owner>  {cm:contactType} [?] A reference to the contact that is responsible for distribution and dissemination of this locations data. See section 6.7

        - <area>  {cm:areaType} [?] A region may represent a geographical area. This is represented in CMML by a closed polygon. Locations inside this polygon belong to the specified area.

        - <route>  {cm:linestringType} [?] A region may represent a route along some path. This is represented in CMML by an open polygon. Locations on the line formed by the polygon belong to the specified route.

      - <station-metadata>   {cm:station-metadataType} [?] Metadata information about a station. At least one child instrument must be present in a <station-metadata> element.

        - <name> {xsd:string} A unique identifier for this location assigned by the client.

        - <description>  {xsd:string} A text description of this location that includes any relevant details that are cannot be further specified in the CMML metadata.

        - <city>  {xsd:string} [?] The city that contains or administers this location. Omit if not applicable.

        - <country>  {cm:ISO-country-codeType} Species the two-letter ISO country code for the country that contains this location.

        - <province>  {cm:CA-province-codeType}  If the location is within Canada, this specifies the two-letter identifier code for the associated province or territory. It should be noted that <state> or <province> must be present in a <station-metadata> element but not both. They are mutually exclusive.

        - <state>  {cm:US-state-codeType} If the location is within the USA, this specifies the two-letter identifier code for the associated state or possession. It should be noted that <state> or <province> must be present in a <station-metadata> element but not both. They are mutually exclusive.

        - <climatic-zone>  {xsd:string} [?] Specifies the climatic zone of this location, if available.

        - <district>  {xsd:string} [?] Specifies the district that contains this location for those clients that group locations in that manner.

        - <timezone>  {cm:timezoneType} Specifies the time zone and time zone offset at this location.

        - <data-owner>  {cm:contactType} A reference to the organization that owns the rights to the data. SEE SECTION 6.7

        - <point>  {cm:pointType} [?] A station is located at a specific geographic point. This includes the latitude, longitude and elevation of the station.

        - <station-state>  {cm:stationStateType} Describes the state of the station. Examples include “active-operational”, “active-testing”, “inactive-uninstalled, “inactive-scheduled-maintenance”, and “inactive-unscheduled-maintenance”.

        - <category>  {cm:station-categoryType} [?] Stations are categorized as “permanent”, “mobile”, or “transportable”.

        - <type>  {cm:station-typeType} [?] Stations are either “automatic”, “staffed” of of “unknown” type.

        - <installation-date>  {xsd:date} [?] The date when this station was installed at this location.

        - <commission-date>  {xsd:date} [?] The date when this station was commissioned.

        - <last-maintenance-date>  {xsd:date} [?] The date when this station was last maintained.

        - <last-inspection-date>  {xsd:date} [?] The date when this station was last inspected.

        - <station-owner>  {cm:contactType} [?] A reference to the contact that owns the station and associated equipment. See section 6.7

        - <station-maintainer>  {cm:contactType} [?] A reference to the contact that is responsible for maintaining this station. See section 6.7

        - <rpu> {cm:rpuType} [?] Describes the station’s Remote Processing Unit (RPU).

          - cmml-index  {xsd:nonNegativeInteger} Because the RPU can return physical status information, it is treated as a sensor-bearing instrument. This requires a CMML index. Since a station can have no more than one RPU, this index should always have a value of “1”. This is a required attribute to offer future flexibility

          - <equipment-information>  {cm:equipment-informationType} [?] SEE SECTION 6.8

          - <ip-address>  {cm:ip-addressType} [?] The IP address of the RPU, if it is connected to the internet.

          - <phone-number>  {cm:phone-numberType} [?] The phone number of the RPU’s internal modem, if available.

          - <physical-status>  {cm:physical-statusSensType} [*] Describes which CMML physical status data the RPU can return.

            - type  {cm:windTypeType} The type of measurement the physical status sensor measures.

            - <description> {xsd:string} [*] A text description of the sensor.

            - <instrument-state> {xsd:string} Current state of the sensor.

            - <units>{cm:unitType} [*] The unit of measurement the sensor is measured in.

            - <accuracy>{xsd:string} [*] The accuracy of the sensor

            - <resolution>{xsd:string} [*] The resolution of the sensor.

            - <sampling-interval>{xsd:string} [*] The time between each sample.

            - <lower-range> {xsd:decimal} [*] The smallest value of the sensor.

            - <upper-range>{xsd:decimal} [*] The largest value of the sensor.

        - <reporting-frequency>  {xsd:string} [?] Describes in words how frequently this station reports data.

        - <vegetation>   {cm:vegetationType} [*] Describes the type of vegetation present at the site.

          - <type> {xsd:string} [?] Describes the type of vegetation.

          - <age> {xsd:string} [?] Indicates the age of the vegetation

          - <height> {xsd:string} [?] Indicates the height of the vegetation

          - <proximity>  {xsd:string} [?] Indicates the proximity of the vegetation to the station

          - <direction>  {cm:direction-codeType} [?] Indicates the direction of the vegetation.

        - <pressure-instrument>  {cm:pressure-instrumentType} [*] Describes the various pressure instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:pressureTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd:gMonthDay} Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd:gMonthDay} Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <wind-instrument>  {cm:wind-instrumentType} [*] Describes the various wind instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:windTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd:gMonthDay} Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd:gMonthDay} Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <temperature-instrument>  {cm:temperature-instrumentType} [*] Describes the various temperature instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:temperatureTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd:gMonthDay} Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd:gMonthDay} Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <precipitation-instrument>  {cm:precipitation-instrumentType} [*] Describes the various precipitation instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:precipitationTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd:gMonthDay} Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd:gMonthDay} Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <radiation-instrument>  {cm:radiation-instrumentType} [*] Describes the various radiation instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:radiationTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd:gMonthDay} Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd:gMonthDay} Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <visibility-instrument>  {cm:visibility-instrumentType} [*] Describes the various visibility instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:visibilityTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd:gMonthDay} Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd:gMonthDay} Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <pavement-instrument>  {cm:pavement-instrumentType} [*] Describes the various pavement instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:pavementTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd:gMonthDay} Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd: gMonthDay } Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <subsurface-instrument>  {cm:subsurface-instrumentType} [*] Describes the various subsurface instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:subsurfaceTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd: gMonthDay } Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd: gMonthDay } Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <air-quality-instrument>  {cm:air-quality-instrumentType} [*] Describes the various air-quality instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:air-qualityTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd: gMonthDay } Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd: gMonthDay } Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <humidity-instrument>  {cm:humidity-instrumentType} [*] Describes the various humidity instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:humidityTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd: gMonthDay } Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd: gMonthDay } Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <snow-instrument>  {cm:snow-instrumentType} [*] Describes the various snow instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:snowTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd: gMonthDay } Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd: gMonthDay } Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <weather-instrument>  {cm:weather-instrumentType} [*] Describes the various weather instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:weatherTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd: gMonthDay } Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd: gMonthDay } Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <snapshot-camera-instrument>  {cm:snapshot-camera-instrumentType} [*] Describes the various snapshot-camera instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - type-  {cm:snapshot-cameraTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd: gMonthDay } Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd: gMonthDay } Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

        - <extension-instrument>  {cm:extension-instrumentType} [*] Describes the various extension instruments associated with this station.

          - cmml-index-  {xsd:nonNegativeInteger} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - sensor-name-  {cm:extensionTypeType} The CMML index is used to distinguish measurements returned by different instruments. This is a required attribute.

          - <description>  {xsd:string} [?] Describes the instrument in detail, including any details deemed relevant by the client that are not captured elsewhere in the CMML metadata.

          - <instrument-state>  {cm:instrumentStateType} Describes the state of the instrument. Examples include “active-operational”, “active-not-reporting”, “inactive-suppressed”, and “inactive-unscheduled-maintenance”.

          - <units>{cm:unitType} [?] The unit of measurement the sensor is measured in.

          - <accuracy>{xsd:string} [?] The accuracy of the sensor

          - <resolution>{xsd:string} [?] The resolution of the sensor.

          - <sampling-interval>{xsd:string} [?] The time between each sample.

          - <lower-range> {xsd:decimal} [?] The smallest value of the sensor.

          - <upper-range>{xsd:decimal} [?] The largest value of the sensor.

          - <instrument-id> {xsd:string} [?] A client assigned identifier for this instrument. This should be sufficient to identify the instrument if the associated station is also known.

          - <point>  {cm:pointType} [?] Specifies the location of an instrument when different than the location of station it is associated with. Omit if not required. see section 6.1.

          - <equipment-information>  {cm:equipment-informationType} [?] see section 6.8

          - <installation-date>  {xsd:date} [?] The date this instrument was installed in its current location.

          - <commission-date>  {xsd:date} [?] The date this instrument was commissioned.

          - <last-maintenance-date>  {xsd:date} [?] The date this instrument was last maintained.

          - <last-inspection-date>  {xsd:date} [?] The date this instrument was last inspected.

          - <calibration-history> {cm:calibration-historyType} [?] Describes when and how this instrument has been calibrated.

            - <calibration>   {cm:calibrationType} [*] Each calibration event is recorded individually.

              - date {xsd:time}  The date when this calibration was performed. This is a required attribute.

              - <calibrated-by>  {cm:contactType} A client assigned identifier for the person or organization responsible for this calibration.

              - <comment>  {cm:calibrationCommentType} [*] Any relevant details pertaining to this calibration.

          - <lane-number>  {xsd:string} [?] A text description of the lane this instrument is located in. This should follow the standard used by the client.

          - <road-construction>  {cm:road-constructionType} [?] Details the construction of the roadway in which a pavement or subsurface sensor is embedded. Omit for other sensors. Must contain exactly one of the following elements: <road>, <bridge>, or <overpass>.

            - <road>  {cm:roadType} The default roadway type. Any roadway that is not a bridge or an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <earthwork>  {cm:earthworkType} [?] How the road is situated relative to surrounding land. The valid results are: “cut” – lower than surrounding land, “fill” – built up relative to surrounding land, or “neutral”.

            - <bridge>  {cm:bridgeType} A bridge must be over water. For CMML metadata purposes, any other raised roadway will be considered an overpass.

              - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the bridge’s construction. The list presently defined is not comprehensive.

              - <water-surface-elevation>  {cm:elevationType} [?] The typical elevation of the water surface beneath the bridge.

            - <overpass>  {cm:overpassType} Any elevated roadway that does not cross over water.

            - <surface>  {cm:surfaceType} The type of material making up the road surface. The list presently defined is not comprehensive.

              - <slope>  {cm:slopeType} [?] The angle, in degrees, that the road makes from the horizontal.

              - <aspect>  {cm:aspectType} [?] The direction in which the slope faces at this location.

              - <pavement-surface-elevation>  {cm:elevationType} [?] The elevation of the pavement surface.

              - <material>  {cm:bridge-or-overpass-materialType} [?] The primary material used in the overpass’s construction. The list presently defined is not comprehensive.

              - <lower-roadway-elevation>  {cm:elevationType} [?] The elevation of the roadway or other surface immediately beneath the overpass.

          - <shadowing>  {cm:shadowingType} [*] The times of day when this instrument will be in shadow at different times of the year.

            - start-date  {xsd: gMonthDay } Specifies the first date at which this shadowing value is valid. This is a required attribute.

            - end-date  {xsd: gMonthDay } Specifies the last date at which this shadowing value is valid. This is a required attribute.

            - <shadowing-starts>  {xsd:time} Specifies the time of day when the instrument is first overcome by shadows.

            - <shadowing-ends>  {xsd:time} Specifies the time of day when the instrument ceases to be in continual shadow.

    - <supplementary-metadata> {cm:supplementary-metadataType} [?] Metadata to help further define the interpretation of observed data. At least one child element of either <conversion-definitions>, <categorical-definitions>, or <sampling-definitions> and one <data-owner> element must be present in a <supplementary-metadata> element.

      - <data-owner>  {cm:contactType} A reference to the organization that owns the rights to the data. SEE SECTION 6.7

      - <categorical-definitions>    {cm:categorical-DefinitionsType} [?] Contains the definitions of categories found in parameter value elements

        - <categorical-table>  {cm:categorical-tableType} [+] Contains the categorical data.

          - category {xsd:string} The category corresponding to the value. This is a required attribute.

          - units {xsd:string} The units of the data in the categories. This is a required attribute.

          - <categorical-key> {xsd:string} The primary key relating the data in the parameter element to a particular categorical table.

          - <value> {cm:valueCategoryType} [+] The value found in the data.

      - <conversion-definitions> {cm:conversion-definitionsType} [?] Contains look-up tables used to relate numerical data to an equivalent value.

        - <conversion-table> {cm:conversion-tableType} [+] Holds the range of data associated with each equivalent value.

          - units {xsd:string} Specifies the units. This is a required attribute.

          - <conversion-key> The primary key relating the data in the parameter element to a particular conversion table.

          - <start-value> {xsd:anyType} The starting value of the range of data for which the value is equivalent.

          - <end-value> {xsd:anyType} The ending value of the range of data for which the value is equivalent.

          - <equivalent-value> {xsd:string} The equivalent value of the data.

      - <sampling-definitions> {cm:sampling-definitionsType} [?] Contains look-up tables used to qualify the values. N.B. : To obtain sampling period you must calculate the following equation: sampling-period = sampling-start + (number-of-samples * sample-interval)

        - <sampling-table> {cm:sampling-tableType} [+] Holds the range of data associated with each equivalent value

          - <sampling-key> The primary key relating the data in the parameter element to a particular sampling table.

          - <sampling-interval>  {cm:sampling-intervalType} The interval between each sample. Must be a non-negative integer number.

            - units {xsd:string} Specifies the time units used to define the sampling interval.

          - <number-of-samples> {xsd:nonNegativeInteger} Specifies the number of samples in the sampling period.

          - <sampling-start> {xsd:nonNegativeInteger} Start of measure into the sampling period in the same units. Example: For a sampling period of 600seconds (10 minutes), if we only take sample from the last 2 minutes the value will be 480.

- Common Element and Attribute Definitions :

  - <point> {cm:pointType} [?]: Element used to define the grid point for which the data is valid.

    - <latitude>  {xsd:decimal}: The latitude of the point where the data is valid.

    - <longitude>  {xsd:decimal}: The longitude of the point where the data is valid.

    - <elevation>  {cm:elevationType}: The elevation of the point where the data is valid.

      - datum {cm:datumType} : A reference point for the elevation. See Table E-7: Valid pavement-forecast datum types.

      - units {cm:elevation-unitsType } : The units of measurement the elevation is in. See Table E-8: Valid pavement-forecast elevation-units types.

  - summarization {cm:summarizationType} [?]: See Table E-19: Valid pavement-forecast summarization types.

    - Type of Spatial Summarization: Collections of grid point values may be summarized into a single value. If this is done, the type of summarization will be provided (mean, median, mode, centroid value etc.).

    - Type of Temporal Summarization: A number of grid point values may be summarized for a given period of time. When this is done, the type of temporal summarization will be provided (period, hourly, 3 hourly, daily, etc.).

    - A value of “none” indicates that the values are valid at a single grid point or time.

  - time-layout {cm: time-layoutAttributeType}: Defines the key to the appropriate valid times and any relevant period name information.

  - <qualifier> {cm:qualifierType}[*]: Element used to hold optional values that describe the parent Element it belongs to.

    - type {cm:qualifiertypeType} : The type of qualifier. Example values include “height”, “name”, “lane-number”, “conversion-table”,”categorical-table”, “sensor-depth”, and “sampling-table”.

    - units {cm:unitType} : Indicates what unit of measurement the qualifier is in.

  - <value> {cm:valueType}: Element used to hold a single observed value

    - units {cm:unitType} : Indicates what unit of measurement the value is in.

  - <qc> {cm:qcType}[*]: Contains quality control elements

    - performer {xsd:string} : Who performed the quality control.

    - <summary> {cm:summaryType }: A summary of the most severe problem with the value observed.

    - <qc-flag> {cm:qc-flagType} [+]: Element used to define the quality control that was carried out on the value and the result of this one.

      - type {cm:qc-flagtypeType}: Specifies the type of quality control that this flag defines. Examples values include “range”, “temporal” and “inter-variable”.

      - value {cm:qc-flagvalueType}: The value resulting from the quality control. Examples values include “doubtful” and “inconsistency”.

      - associated-measurement-category {cm:measurement-categoryType} [?]: The parameter that is associated with the current measurement when quality control type is "inter-variable".

      - associated-measurement-type { cm:measurement-typeType } [?]: The parameter type that is associated with the current measurement types when quality control type is "inter-variable".

      - <message> {xsd:messageType} [+]: Element that contains the quality control message.

        - language {xsd:string} [?]: The language of the message. Examples values include “en”and “fr”. The default value is “en”.

  - <contact>{cm:contactType} A person or organization that can be contacted for additional information about certain designated areas of responsibility.

    - <name> {xsd:string} The name of the contact or contact organization.

    - <address> {xsd:string} [?] A mailing address for this contact.

    - <phone-number>{cm:phone-numberType} [?] A phone number where this contact can be reached.

    - <email> {cm:emailType} [?] An email address where this contact can be reached.

  - <equipment-information>{cm:equipmentType} Each significant piece of equipment associated with a station needs to have certain common values recorded, if they are available. Significant pieces of equipment include the RPU and all measuring instruments.

    - <manufacturer> {xsd:string} [?] Specifies the name of the manufacturer of this piece of equipment.

    - <model-number> {xsd:string} [?]Specifies the model number of this piece of equipment, if available.

    - <serial-number> {xsd:string} [?] Specifies the serial number of this piece of equipment, if available.

  - <exception>:  An exception or local effect is defined as the occurrence of a weather element specific to a location/time inside a forecast region. The exception generally refers to a particular geographical location in the forecast region (Along the coast, Over higher terrain, Inland …). The exception statement is attached to the main weather element or it can stand alone if no main condition exist but some weather element is forecasted as an exception.  The exception value will be kept constant for the duration of weather element to which it is related. Thus, exceptions will not be interpolated, they will have a constant value for a define duration. Main weather elements (not exceptions) can be interpolated. Only the following elements for meteocode-forecast can have exceptions:  cloud, precipitation, temperature, wind, visibility and freezing spray. They are defined as {cm:scribe-cloud-cover-exceptionType}, {cm:scribe-precipitation-event-exceptionType}, {cm:scribe-temperature-value-exceptionType}, {cm:scribe-wind-exceptionType}, {cm:scribe-visibilty-exceptionType} and {cm:scribe-freezing-spray-exceptionType} respectively.

  - <excep-category> {cm:scribe-excep-categoryType}:

    - excep-type {cm:scribe-excep-typeType}:  See Table E-28: Valid values for meteocode-forecast exception types.

    - excep-code {cm:scribe-excep-codeType}:  See Table E-27: Valid values for meteocode-forecast exception codes.

    - <excep-description> {cm:scribe-excep-discriptionType}:   Description of the exception.

      - lang {cm:scribe-langType}: Language. Values are “en” for English or “fr” for French.

- XML Considerations .

  - Namespace: CMML will use the namespace http://www_todefine_com

  - Schema: CMML will use XML Schema to perform validity checking.

  - Character Set: CMML will use ISO-8859-1 encoding.

- Requirements

The Road Weather Information System for Canada (RWISC) is being implemented to provide a more efficient, sustainable and safer highway system. This is achieved by integrating a Canada-wide network of Environmental System Sensors (ESS) and instrumented vehicles which provide a continuous stream of observation data describing road conditions. Provincial Computer Centers acquire data from fixed and mobile ESS stations in real-time.

In order to facilitate this integration, a requirement was identified to develop a standard way of exchanging data between Canadian provincial servers and federal government servers. By leveraging standards, RWISC will be able to tie together disparate information from multiple heterogeneous systems nationally and facilitate information transfer between various agencies to support their needs.

After reviewing the strengths of NOAA’s Digital Weather Markup Language (DWML) and the Road Web Markup Language (RWML) from Japan, it was decided to adopt DWML as the basis for a new XML schema standard to facilitate data exchange between diversified systems. DWML is not ideally suited for moving road weather information and it required major modifications. As a result, the Canadian Meteorological Markup Language (CMML) evolved.

CMML is an XML language for the encoding and transfer of weather and road weather data to standardize data transfer between RWIN and Provincial Computer Centers. The CMML specification is being written with enough flexibility to accommodate other environmental science and sensor applications. The CMML element definitions are based on the National Transportation Communications for ITS Protocol (NTCIP) Centre to Field standard (NTCIP 1204), and World Meteorological Organization (WMO) standards. By providing general flexibility and ease of maintenance, emphasis on industry standards minimizes development efforts and encourages interaction with other decision support systems.

- Meteocode-Forecast Requirements

The Storm Prediction Centers of Canada currently produce Canadian weather forecasts using the Scribe forecast product application. These forecasts are available in two formats.  The first one is a text format to which all Canadian are used to. The second one is a digital format called meteocode in which all weather elements are described explicitly. In fact, the text format is generated from the digital format and constitute a generalization of the detailed weather elements suite contained in the digital format. The digital format has now been translated in XML format for the benefit of any user that would like to extract and format, partially or totally, any of the official Weather Elements forecasted by Environment Canada.

- Structure and Type Definitions

Complex Types	C-6

air-qualityInsType	C-6

air-qualityObsType	C-8

air-qualityType	C-9

areaType	C-9

base-metadataType	C-10

basic-instrumentType	C-11

bridgeType	C-12

calibration-historyType	C-12

calibrationType	C-12

categorical-tableType	C-13

circleType	C-13

cityType	C-13

cmmlType	C-13

contactType	C-14

conversion-tableType	C-14

creation-dateType	C-14

dataType	C-15

elevationType	C-15

equipment-informationType	C-15

extensionInsType	C-16

extensionType	C-18

forecast-locationType	C-19

forecastType	C-19

headType	C-19

humidityInsType	C-20

humidityObsType	C-22

humidityType	C-23

identifierType	C-23

instrumentType	C-24

linestringType	C-26

location-metadataType	C-26

locationType	C-26

measurementType	C-27

messageType	C-27

metadataType	C-27

nws-zoneType	C-27

observation-seriesType	C-28

observationType	C-29

originType	C-30

overpassType	C-30

parametersType	C-31

pavement-forecastType	C-32

pavementInsType	C-33

pavementObsType	C-35

pavementType	C-36

physical-statusInsType	C-37

physical-statusObsType	C-38

physical-statusType	C-39

pointType	C-39

polygonType	C-39

precipitationInsType	C-40

precipitationObsType	C-42

precipitationType	C-43

pressureInsType	C-44

pressureObsType	C-46

pressureType	C-47

production-centerType	C-47

productType	C-48

qc-flagType	C-49

qcType	C-49

qualifierType	C-49

radiationInsType	C-50

radiationObsType	C-52

radiationType	C-53

radiusType	C-53

rectangleType	C-53

region-metadataType	C-54

road-constructionType	C-55

roadType	C-55

roadwayType	C-55

rpuType	C-56

sampling-tableType	C-56

scribe-accum-amountType	C-57

scribe-accum-listType	C-57

scribe-accum-lower-limitType	C-57

scribe-accum-upper-limitType	C-57

scribe-amended-regionsType	C-58

scribe-basic-cloud-coverType	C-58

scribe-basic-exceptionType	C-58

scribe-basic-freezing-sprayType	C-59

scribe-basic-precipitation-eventType	C-59

scribe-basic-visibilityType	C-60

scribe-basic-windType	C-60

scribe-cloud-cover-exceptionType	C-61

scribe-cloud-coverType	C-61

scribe-cloud-listType	C-62

scribe-excep-categoryType	C-62

scribe-forecastType	C-62

scribe-freezing-spray-exceptionType	C-63

scribe-freezing-spray-listType	C-63

scribe-freezing-sprayType	C-63

scribe-gust-speedType	C-64

scribe-ice-cover-listType	C-64

scribe-ice-coverType	C-64

scribe-locationType	C-64

scribe-msc-zone-codeType	C-65

scribe-msc-zone-nameType	C-65

scribe-parametersType	C-66

scribe-pointType	C-66

scribe-precipitation-event-exceptionType	C-67

scribe-precipitation-eventType	C-67

scribe-precipitation-listType	C-68

scribe-probability-of-precipitation-listType	C-68

scribe-probability-of-precipitationType	C-68

scribe-snow-level-listType	C-68

scribe-snow-levelType	C-69

scribe-temperature-listType	C-69

scribe-temperature-value-exceptionType	C-69

scribe-temperature-valueType	C-70

scribe-UV-index-listType	C-70

scribe-valueType	C-70

scribe-visibility-exceptionType	C-71

scribe-visibility-listType	C-71

scribe-visibilityType	C-72

scribe-warning-eventType	C-72

scribe-warning-listType	C-73

scribe-wave-height-listType	C-73

scribe-wave-heightType	C-73

scribe-wind-exceptionType	C-74

scribe-wind-listType	C-74

scribe-wind-speedType	C-74

scribe-windType	C-75

shadowingType	C-75

snapshot-cameraInsType	C-76

snapshot-cameraObsType	C-78

snowInsType	C-79

snowObsType	C-81

snowType	C-82

sourceType	C-83

start-valid-timeType	C-83

station-metadataType	C-84

subsurfaceInsType	C-87

subsurfaceObsType	C-89

subsurfaceType	C-90

supplementary-metadataType	C-91

temperatureInsType	C-92

temperatureObsType	C-94

temperatureType	C-95

time-layoutElementType	C-96

timezoneType	C-96

valueCategoryType	C-96

valueType	C-96

vegetationType	C-97

visibilityInsType	C-98

visibilityObsType	C-100

visibilityType	C-101

weatherInsType	C-102

weatherObsType	C-104

weatherType	C-105

windInsType	C-106

windObsType	C-108

windType	C-109

Simple Types	C-109

air-qualityTypeType	C-109

bridge-or-overpass-materialType	C-109

calibrationCommentType	C-109

CA-province-codeType	C-110

categoryType	C-110

datumType	C-110

direction-codeType	C-110

earthworkType	C-110

elevation-unitsType	C-110

emailType	C-110

fieldType	C-110

humidityTypeType	C-110

identifiertypeType	C-110

instrumentStateType	C-110

ip-addressType	C-110

ISO-country-codeType	C-110

layout-keyType	C-110

measurement-categoryType	C-110

measurement-typeType	C-111

non-emptyString	C-111

offsetType	C-111

operational-modeType	C-111

origintypeType	C-111

pavementTypeType	C-111

phone-numberType	C-111

physical-statusTypeType	C-111

precipitationTypeType	C-111

pressureTypeType	C-111

provinceType	C-111

qc-flagtypeType	C-111

qc-flagvalueType	C-111

radiationTypeType	C-111

radius-unitsType	C-112

scribe-accum-typeType	C-112

scribe-accum-unitsType	C-112

scribe-basic-simple-cloud-coverType	C-112

scribe-cloud-cover-ceiling-codeType	C-112

scribe-cloud-unitsType	C-112

scribe-excep-codeType	C-112

scribe-excep-typeType	C-112

scribe-freezing-spray-frequencyType	C-112

scribe-freezing-spray-ice-cover-modifierType	C-112

scribe-freezing-spray-intensityType	C-112

scribe-freezing-spray-occurType	C-112

scribe-langType	C-112

scribe-msc-zone-code-statusType	C-112

scribe-precipitation-frequencyType	C-112

scribe-precipitation-intensityType	C-113

scribe-precipitation-occurType	C-113

scribe-precipitation-typeType	C-113

scribe-simpe-probabilityType	C-113

scribe-snow-level-unitsType	C-113

scribe-statusType	C-113

scribe-temperature-ground-frostType	C-113

scribe-temperature-trendType	C-113

scribe-temperature-typeType	C-113

scribe-temperature-unitsType	C-113

scribe-typeType	C-113

scribe-value-unitsType	C-113

scribe-visibility-frequencyType	C-113

scribe-visibility-typeType	C-113

scribe-warning-codeType	C-114

scribe-warning-statusType	C-114

scribe-warning-typeType	C-114

scribe-wave-height-ice-cover-modifierType	C-114

scribe-wave-height-unitsType	C-114

scribe-wind-directionType	C-114

scribe-wind-modificatorsType	C-114

scribe-wind-unitsType	C-114

slopeType	C-114

snapshot-cameraTypeType	C-114

snowTypeType	C-114

stateType	C-114

station-categoryType	C-114

stationStateType	C-114

station-typeType	C-114

subsurfaceTypeType	C-115

summarizationType	C-115

summaryType	C-115

surfaceType	C-115

temperatureTypeType	C-115

time-coordinateType	C-115

time-layoutAttributeType	C-115

timeUnitsType	C-115

timezonetypeType	C-115

unitType	C-115

US-state-codeType	C-115

visibilityTypeType	C-115

weatherTypeType	C-115

windTypeType	C-116

Complex Types

air-qualityInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/air-quality-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; air-qualityTypeType; required. |

air-qualityObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/air-quality. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; air-qualityTypeType; required. |

air-qualityType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/air-quality. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; air-qualityTypeType; required. height; xsd:nonNegativeInteger; optional. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

areaType

| diagram |  |
| --- | --- |
| children | circle rectangle polygon |
| used by | elements; locationType/area forecast-locationType/area region-metadataType/area. |

base-metadataType

| diagram |  |
| --- | --- |
| children | name description city country province state climatic-zone district timezone data-owner |
| used by | complexTypes; region-metadataType station-metadataType. |

basic-instrumentType

| diagram |  |
| --- | --- |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range |
| used by | complexTypes; instrumentType physical-statusInsType. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. |

bridgeType

| diagram |  |
| --- | --- |
| type | extension of roadwayType |
| properties | base; roadwayType. |
| children | surface slope aspect pavement-surface-elevation material water-surface-elevation |
| used by | element; road-constructionType/bridge. |

calibration-historyType

| diagram |  |
| --- | --- |
| children | calibration |
| used by | element; instrumentType/calibration-history. |

calibrationType

| diagram |  |
| --- | --- |
| children | calibrated-by comment |
| used by | element; calibration-historyType/calibration. |
| attributes | Name; Type; Use; Default; Fixed. date; xsd:date; required. |

categorical-tableType

| diagram |  |
| --- | --- |
| children | categorical-key value |
| used by | element; supplementary-metadataType/categorical-definitions/categorical-table. |
| attributes | Name; Type; Use; Default; Fixed. units; unitType; required. |

circleType

| diagram |  |
| --- | --- |
| children | point radius |
| used by | element; areaType/circle. |

cityType

| diagram |  |
| --- | --- |
| type | extension of xsd:string |
| properties | base; xsd:string. |
| used by | element; forecast-locationType/city. |
| attributes | Name; Type; Use; Default; Fixed. province; provinceType; required. summarization; summarizationType; optional. |

cmmlType

| diagram |  |
| --- | --- |
| children | head data |
| used by | element; cmml. |
| attributes | Name; Type; Use; Default; Fixed. version; non-emptyString; required. |

contactType

| diagram |  |
| --- | --- |
| children | name address phone-number email |
| used by | elements; calibrationType/calibrated-by supplementary-metadataType/data-owner base-metadataType/data-owner station-metadataType/station-maintainer station-metadataType/station-owner. |

conversion-tableType

| diagram |  |
| --- | --- |
| children | conversion-key start-value end-value equivalent-value |
| used by | element; supplementary-metadataType/conversion-definitions/conversion-table. |
| attributes | Name; Type; Use; Default; Fixed. units; unitType; required. |

creation-dateType

| diagram |  |
| --- | --- |
| type | extension of xsd:dateTime |
| properties | base; xsd:dateTime. |
| used by | element; productType/creation-date. |
| attributes | Name; Type; Use; Default; Fixed. refresh-frequency; xsd:duration; required. |

dataType

| diagram |  |
| --- | --- |
| children | metadata observation-series forecast |
| used by | element; cmmlType/data. |

elevationType

| diagram |  |
| --- | --- |
| type | extension of xsd:decimal |
| properties | base; xsd:decimal. |
| used by | elements; pointType/elevation overpassType/lower-roadway-elevation roadwayType/pavement-surface-elevation subsurfaceInsType/sensor-depth bridgeType/water-surface-elevation. |
| attributes | Name; Type; Use; Default; Fixed. datum; datumType; required. units; elevation-unitsType; required. |

equipment-informationType

| diagram |  |
| --- | --- |
| children | manufacturer model-number serial-number |
| used by | elements; rpuType/equipment-information instrumentType/equipment-information. |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/extension-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. sensor-name; xsd:string; required. |

extensionInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/extension-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. sensor-name; xsd:string; required. |

extensionType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/extension. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; xsd:string; required. height; xsd:nonNegativeInteger; optional. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

forecast-locationType

| diagram |  |
| --- | --- |
| children | location-key point city nws-zone area station route |

forecastType

| diagram |  |
| --- | --- |
| children | meteocode-forecast pavement-forecast |
| used by | element; dataType/forecast. |

headType

| diagram |  |
| --- | --- |
| children | product source |
| used by | element; cmmlType/head. |

humidityInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/humidity-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; humidityTypeType; required. |

humidityObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/humidity. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; humidityTypeType; required. |

humidityType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/humidity. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; humidityTypeType; required. height; xsd:nonNegativeInteger; optional. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

identifierType

| diagram |  |
| --- | --- |
| properties | mixed; true. |
| used by | element; originType/id. |
| attributes | Name; Type; Use; Default; Fixed. type; identifiertypeType; required. |

instrumentType

| diagram |  |
| --- | --- |
| type | extension of basic-instrumentType |
| properties | base; basic-instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | complexTypes; air-qualityInsType extensionInsType humidityInsType pavementInsType precipitationInsType pressureInsType radiationInsType snapshot-cameraInsType snowInsType subsurfaceInsType temperatureInsType visibilityInsType weatherInsType windInsType. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. |

linestringType

| diagram |  |
| --- | --- |
| children | point |
| used by | elements; locationType/route forecast-locationType/route region-metadataType/route. |

location-metadataType

| diagram |  |
| --- | --- |
| type | extension of originType |
| properties | base; originType. |
| children | id station-metadata region-metadata |
| used by | element; metadataType/location. |
| attributes | Name; Type; Use; Default; Fixed. type; origintypeType; required. |

locationType

| diagram |  |
| --- | --- |
| children | point area route |
| used by | elements; pavement-forecastType/location observation-seriesType/location. |

measurementType

| diagram |  |
| --- | --- |
| children | qualifier value qc |
| used by | element; observationType/extension. complexTypes; air-qualityObsType humidityObsType pavementObsType physical-statusObsType precipitationObsType pressureObsType radiationObsType snapshot-cameraObsType snowObsType subsurfaceObsType temperatureObsType visibilityObsType weatherObsType windObsType. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. |

messageType

| diagram |  |
| --- | --- |
| properties | mixed; true. |
| used by | element; qc-flagType/message. |
| attributes | Name; Type; Use; Default; Fixed. language; required. |

metadataType

| diagram |  |
| --- | --- |
| children | location supplementary-metadata |
| used by | element; dataType/metadata. |

nws-zoneType

| diagram |  |
| --- | --- |
| type | extension of xsd:string |
| properties | base; xsd:string. |
| used by | element; forecast-locationType/nws-zone. |
| attributes | Name; Type; Use; Default; Fixed. province; provinceType; required. summarization; summarizationType; optional. |

observation-seriesType

| diagram |  |
| --- | --- |
| children | origin location observation |
| used by | element; dataType/observation-series. |

observationType

| diagram |  |
| --- | --- |
| children | pressure wind temperature precipitation radiation visibility pavement subsurface air-quality physical-status humidity weather snow snapshot-camera extension |
| used by | element; observation-seriesType/observation. |
| attributes | Name; Type; Use; Default; Fixed. valid-time; xsd:dateTime; required. |

originType

| diagram |  |
| --- | --- |
| children | id |
| used by | elements; observation-seriesType/origin forecast-locationType/station. complexType; location-metadataType. |
| attributes | Name; Type; Use; Default; Fixed. type; origintypeType; required. |

overpassType

| diagram |  |
| --- | --- |
| type | extension of roadwayType |
| properties | base; roadwayType. |
| children | surface slope aspect pavement-surface-elevation material lower-roadway-elevation |
| used by | element; road-constructionType/overpass. |

parametersType

| diagram |  |
| --- | --- |
| children | pressure wind temperature precipitation radiation visibility pavement subsurface snow humidity air-quality weather physical-status extension |
| used by | element; pavement-forecastType/parameters. |
| attributes | Name; Type; Use; Default; Fixed. applicable-location; non-emptyString; required. |

pavement-forecastType

| diagram |  |
| --- | --- |
| children | location time-layout parameters |
| used by | element; forecastType/pavement-forecast. |

pavementInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/pavement-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; pavementTypeType; required. |

pavementObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/pavement. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; pavementTypeType; required. |

pavementType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/pavement. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. lane-number; xsd:nonNegativeInteger; optional. type; pavementTypeType; required. height; xsd:integer; optional. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

physical-statusInsType

| diagram |  |
| --- | --- |
| type | extension of basic-instrumentType |
| properties | base; basic-instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range |
| used by | element; rpuType/physical-status. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; physical-statusTypeType; required. |

physical-statusObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/physical-status. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; physical-statusTypeType; required. |

physical-statusType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/physical-status. |
| attributes | Name; Type; Use; Default; Fixed. type; physical-statusTypeType; required. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

pointType

| diagram |  |
| --- | --- |
| children | latitude longitude elevation |
| used by | elements; locationType/point forecast-locationType/point linestringType/point circleType/point rectangleType/point polygonType/point station-metadataType/point instrumentType/point. |

polygonType

| diagram |  |
| --- | --- |
| children | point |
| used by | element; areaType/polygon. |

precipitationInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/precipitation-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; precipitationTypeType; required. |

precipitationObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/precipitation. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; precipitationTypeType; required. |

precipitationType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/precipitation. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; precipitationTypeType; required. units; unitType; required. categorical-code; xsd:string; optional. height; xsd:nonNegativeInteger; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

pressureInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/pressure-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; pressureTypeType; required. |

pressureObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/pressure. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; pressureTypeType; required. |

pressureType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/pressure. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; pressureTypeType; required. units; unitType; required. categorical-code; xsd:string; optional. height; xsd:nonNegativeInteger; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

production-centerType

| diagram |  |
| --- | --- |
| properties | mixed; true. |
| children | sub-center |
| used by | element; sourceType/production-center. |

productType

| diagram |  |
| --- | --- |
| children | title field category creation-date current-issue valid-begin-time valid-end-time next-issue timezone type initials amended-regions status |
| used by | element; headType/product. |
| attributes | Name; Type; Use; Default; Fixed. operational-mode; operational-modeType; required. |

qc-flagType

| diagram |  |
| --- | --- |
| children | message |
| used by | element; qcType/qc-flag. |
| attributes | Name; Type; Use; Default; Fixed. type; qc-flagtypeType; required. value; qc-flagvalueType; required. associated-measurement-category; measurement-categoryType; optional. associated-measurement-type; measurement-typeType; optional. |

qcType

| diagram |  |
| --- | --- |
| children | summary qc-flag |
| used by | element; measurementType/qc. |
| attributes | Name; Type; Use; Default; Fixed. performer; non-emptyString; required. |

qualifierType

| diagram |  |
| --- | --- |
| properties | mixed; true. |
| used by | element; measurementType/qualifier. |
| attributes | Name; Type; Use; Default; Fixed. type; required. units; unitType; required. |

radiationInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/radiation-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; radiationTypeType; required. |

radiationObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/radiation. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; radiationTypeType; required. |

radiationType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/radiation. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; radiationTypeType; required. height; xsd:nonNegativeInteger; optional. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

radiusType

| diagram |  |
| --- | --- |
| type | extension of xsd:decimal |
| properties | base; xsd:decimal. |
| used by | element; circleType/radius. |
| attributes | Name; Type; Use; Default; Fixed. radius-units; radius-unitsType; required. |

rectangleType

| diagram |  |
| --- | --- |
| children | point |
| used by | element; areaType/rectangle. |

region-metadataType

| diagram |  |
| --- | --- |
| type | extension of base-metadataType |
| properties | base; base-metadataType. |
| children | name description city country province state climatic-zone district timezone data-owner area route |
| used by | element; location-metadataType/region-metadata. |

road-constructionType

| diagram |  |
| --- | --- |
| children | road bridge overpass |
| used by | element; instrumentType/road-construction. |

roadType

| diagram |  |
| --- | --- |
| type | extension of roadwayType |
| properties | base; roadwayType. |
| children | surface slope aspect pavement-surface-elevation earthwork |
| used by | element; road-constructionType/road. |

roadwayType

| diagram |  |
| --- | --- |
| children | surface slope aspect pavement-surface-elevation |
| used by | complexTypes; bridgeType overpassType roadType. |

rpuType

| diagram |  |
| --- | --- |
| children | equipment-information ip-address phone-number physical-status |
| used by | element; station-metadataType/rpu. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. |

sampling-tableType

| diagram |  |
| --- | --- |
| children | sampling-key sampling-interval number-of-samples sampling-start |
| used by | element; supplementary-metadataType/sampling-definitions/sampling-table. |

scribe-accum-amountType

| diagram |  |
| --- | --- |
| children | lower-limit upper-limit |
| used by | element; scribe-accum-listType/accum-amount. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. type; scribe-accum-typeType; required. |

scribe-accum-listType

| diagram |  |
| --- | --- |
| children | accum-amount |
| used by | element; scribe-parametersType/accum-list. |
| attributes | Name; Type; Use; Default; Fixed. units; scribe-accum-unitsType; mm. |

scribe-accum-lower-limitType

| diagram |  |
| --- | --- |
| type | extension of xsd:decimal |
| properties | base; xsd:decimal. |
| used by | element; scribe-accum-amountType/lower-limit. |
| attributes | Name; Type; Use; Default; Fixed. cumul-inf; xsd:decimal; optional. |

scribe-accum-upper-limitType

| diagram |  |
| --- | --- |
| type | extension of xsd:decimal |
| properties | base; xsd:decimal. |
| used by | element; scribe-accum-amountType/upper-limit. |
| attributes | Name; Type; Use; Default; Fixed. cumul-sup; xsd:decimal; optional. |

scribe-amended-regionsType

| diagram |  |
| --- | --- |
| children | location |
| used by | element; productType/amended-regions. |

scribe-basic-cloud-coverType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-simple-cloud-coverType |
| properties | base; scribe-basic-simple-cloud-coverType. |
| used by | element; scribe-cloud-listType/cloud-cover. complexType; scribe-cloud-coverType. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. ceiling-code; scribe-cloud-cover-ceiling-codeType; optional. |

scribe-basic-exceptionType

| diagram |  |
| --- | --- |
| children | excep-category |
| used by | complexTypes; scribe-cloud-cover-exceptionType scribe-freezing-spray-exceptionType scribe-precipitation-event-exceptionType scribe-temperature-value-exceptionType scribe-visibility-exceptionType scribe-wind-exceptionType. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-basic-freezing-sprayType

| diagram |  |
| --- | --- |
| used by | element; scribe-freezing-spray-exceptionType/freezing-spray. complexType; scribe-freezing-sprayType. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. frequency; scribe-freezing-spray-frequencyType; required. intensity; scribe-freezing-spray-intensityType; required. occurrence; scribe-freezing-spray-occurType; required. ice-cover-modifier; scribe-freezing-spray-ice-cover-modifierType; optional. |

scribe-basic-precipitation-eventType

| diagram |  |
| --- | --- |
| used by | element; scribe-precipitation-event-exceptionType/precipitation-event. complexType; scribe-precipitation-eventType. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. frequency; scribe-precipitation-frequencyType; required. intensity; scribe-precipitation-intensityType; required. occurrence; scribe-precipitation-occurType; required. type; scribe-precipitation-typeType; required. |

scribe-basic-visibilityType

| diagram |  |
| --- | --- |
| children | value |
| used by | element; scribe-visibility-exceptionType/visibility. complexType; scribe-visibilityType. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. frequency; scribe-visibility-frequencyType; required. type; scribe-visibility-typeType; required. |

scribe-basic-windType

| diagram |  |
| --- | --- |
| children | wind-speed gust-speed |
| used by | element; scribe-wind-exceptionType/wind. complexType; scribe-windType. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. direction; scribe-wind-directionType; required. modificator; scribe-wind-modificatorsType; optional. |

scribe-cloud-cover-exceptionType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-exceptionType |
| properties | base; scribe-basic-exceptionType. |
| children | excep-category cloud-cover |
| used by | element; scribe-cloud-listType/exception. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-cloud-coverType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-cloud-coverType |
| properties | base; scribe-basic-cloud-coverType. |
| used by | element; scribe-cloud-cover-exceptionType/cloud-cover. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. ceiling-code; scribe-cloud-cover-ceiling-codeType; optional. cover-start; xsd:nonNegativeInteger; required. cover-end; xsd:nonNegativeInteger; required. |

scribe-cloud-listType

| diagram |  |
| --- | --- |
| children | cloud-cover exception |
| used by | element; scribe-parametersType/cloud-list. |
| attributes | Name; Type; Use; Default; Fixed. units; scribe-cloud-unitsType; deci. |

scribe-excep-categoryType

| diagram |  |
| --- | --- |
| children | excep-description |
| used by | element; scribe-basic-exceptionType/excep-category. |
| attributes | Name; Type; Use; Default; Fixed. excep-type; scribe-excep-typeType; required. excep-code; scribe-excep-codeType; required. |

scribe-excep-descriptionType

| diagram |  |
| --- | --- |
| type | extension of xsd:string |
| properties | base; xsd:string. |
| used by | element; scribe-excep-categoryType/excep-description. |
| attributes | Name; Type; Use; Default; Fixed. lang; scribe-langType; required. |

scribe-forecastType

| diagram |  |
| --- | --- |
| children | location parameters |
| used by | element; forecastType/meteocode-forecast. |

scribe-freezing-spray-exceptionType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-exceptionType |
| properties | base; scribe-basic-exceptionType. |
| children | excep-category freezing-spray |
| used by | element; scribe-freezing-sprayType/exception. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-freezing-spray-listType

| diagram |  |
| --- | --- |
| children | freezing-spray |
| used by | element; scribe-parametersType/freezing-spray-list. |

scribe-freezing-sprayType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-freezing-sprayType |
| properties | base; scribe-basic-freezing-sprayType. |
| children | exception |
| used by | element; scribe-freezing-spray-listType/freezing-spray. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. frequency; scribe-freezing-spray-frequencyType; required. intensity; scribe-freezing-spray-intensityType; required. occurrence; scribe-freezing-spray-occurType; required. ice-cover-modifier; scribe-freezing-spray-ice-cover-modifierType; optional. |

scribe-gust-speedType

| diagram |  |
| --- | --- |
| children | lower-limit upper-limit |
| used by | element; scribe-basic-windType/gust-speed. |

scribe-ice-cover-listType

| diagram |  |
| --- | --- |
| children | ice-cover |
| used by | element; scribe-parametersType/ice-cover-list. |
| attributes | Name; Type; Use; Default; Fixed. units; xsd:string; required; %. |

scribe-ice-coverType

| diagram |  |
| --- | --- |
| type | extension of scribe-simple-probabilityType |
| properties | base; scribe-simple-probabilityType. |
| used by | element; scribe-ice-cover-listType/ice-cover. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-locationType

| diagram |  |
| --- | --- |
| children | msc-zone-code point msc-zone-name |
| used by | elements; scribe-amended-regionsType/location scribe-forecastType/location. |

scribe-msc-zone-codeType

| diagram |  |
| --- | --- |
| type | extension of xsd:string |
| properties | base; xsd:string. |
| used by | element; scribe-locationType/msc-zone-code. |
| attributes | Name; Type; Use; Default; Fixed. status; scribe-msc-zone-code-statusType. |

scribe-msc-zone-nameType

| diagram |  |
| --- | --- |
| type | extension of xsd:string |
| properties | base; xsd:string. |
| used by | element; scribe-locationType/msc-zone-name. |
| attributes | Name; Type; Use; Default; Fixed. lang; scribe-langType; required. |

scribe-parametersType

warning-list cloud-list precipitation-list probability-of-precipitation-list accum-list snow-level-list temperature-list wind-list visibility-list UV-index-list freezing-spray-list wave-height-list ice-cover-list

element; scribe-forecastType/parameters.

scribe-pointType

| diagram |  |
| --- | --- |
| children | latitude longitude |
| used by | element; scribe-locationType/point. |

scribe-precipitation-event-exceptionType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-exceptionType |
| properties | base; scribe-basic-exceptionType. |
| children | excep-category precipitation-event |
| used by | element; scribe-precipitation-eventType/exception. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-precipitation-eventType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-precipitation-eventType |
| properties | base; scribe-basic-precipitation-eventType. |
| children | exception precipitation-event |
| used by | elements; scribe-precipitation-listType/precipitation-event scribe-precipitation-eventType/precipitation-event. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. frequency; scribe-precipitation-frequencyType; required. intensity; scribe-precipitation-intensityType; required. occurrence; scribe-precipitation-occurType; required. type; scribe-precipitation-typeType; required. |

scribe-precipitation-listType

| diagram |  |
| --- | --- |
| children | precipitation-event |
| used by | element; scribe-parametersType/precipitation-list. |

scribe-probability-of-precipitation-listType

| diagram |  |
| --- | --- |
| children | probability-of-precipitation |
| used by | element; scribe-parametersType/probability-of-precipitation-list. |
| attributes | Name; Type; Use; Default; Fixed. units; xsd:string; required; %. |

scribe-probability-of-precipitationType

| diagram |  |
| --- | --- |
| type | extension of scribe-simple-probabilityType |
| properties | base; scribe-simple-probabilityType. |
| used by | element; scribe-probability-of-precipitation-listType/probability-of-precipitation. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-snow-level-listType

| diagram |  |
| --- | --- |
| children | snow-level |
| used by | element; scribe-parametersType/snow-level-list. |
| attributes | Name; Type; Use; Default; Fixed. units; scribe-snow-level-unitsType; m. |

scribe-snow-levelType

| diagram |  |
| --- | --- |
| type | extension of xsd:nonNegativeInteger |
| properties | base; xsd:nonNegativeInteger. |
| used by | element; scribe-snow-level-listType/snow-level. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-temperature-listType

| diagram |  |
| --- | --- |
| children | temperature-value exception |
| used by | element; scribe-parametersType/temperature-list. |
| attributes | Name; Type; Use; Default; Fixed. units; scribe-temperature-unitsType; celsius. type; scribe-temperature-typeType; required. |

scribe-temperature-value-exceptionType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-exceptionType |
| properties | base; scribe-basic-exceptionType. |
| children | excep-category temperature-value |
| used by | element; scribe-temperature-listType/exception. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-temperature-valueType

| diagram |  |
| --- | --- |
| children | limit lower-limit upper-limit |
| used by | elements; scribe-temperature-listType/temperature-value scribe-temperature-value-exceptionType/temperature-value. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. trend; scribe-temperature-trendType. ground-frost; scribe-temperature-ground-frostType. |

scribe-UV-index-listType

| diagram |  |
| --- | --- |
| children | UV-index |
| used by | element; scribe-parametersType/UV-index-list. |

scribe-valueType

| diagram |  |
| --- | --- |
| type | extension of xsd:decimal |
| properties | base; xsd:decimal. |
| used by | element; scribe-basic-visibilityType/value. |
| attributes | Name; Type; Use; Default; Fixed. units; scribe-value-unitsType; NM. |

scribe-UV-indexType

| diagram |  |
| --- | --- |
| type | extension of xsd:decimal |
| properties | base; xsd:decimal. |
| used by | element; scribe-UV-index-listType/UV-index. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-visibility-exceptionType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-exceptionType |
| properties | base; scribe-basic-exceptionType. |
| children | excep-category visibility |
| used by | element; scribe-visibilityType/exception. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-visibility-listType

| diagram |  |
| --- | --- |
| children | visibility |
| used by | element; scribe-parametersType/visibility-list. |

scribe-visibilityType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-visibilityType |
| properties | base; scribe-basic-visibilityType. |
| children | value exception |
| used by | element; scribe-visibility-listType/visibility. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. frequency; scribe-visibility-frequencyType; required. type; scribe-visibility-typeType; required. |

scribe-warning-eventType

| diagram |  |
| --- | --- |
| children | warning-event |
| used by | elements; scribe-warning-listType/warning-event scribe-warning-eventType/warning-event. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. type; scribe-warning-typeType; required. code; scribe-warning-codeType; required. status; scribe-warning-statusType; required. |

scribe-warning-listType

| diagram |  |
| --- | --- |
| children | warning-event |
| used by | element; scribe-parametersType/warning-list. |

scribe-wave-height-listType

| diagram |  |
| --- | --- |
| children | wave-height |
| used by | element; scribe-parametersType/wave-height-list. |
| attributes | Name; Type; Use; Default; Fixed. units; scribe-wave-height-unitsType; m. |

scribe-wave-heightType

| diagram |  |
| --- | --- |
| children | lower-limit upper-limit |
| used by | element; scribe-wave-height-listType/wave-height. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. ice-cover-modifier; scribe-wave-height-ice-cover-modifierType; optional. |

scribe-wind-exceptionType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-exceptionType |
| properties | base; scribe-basic-exceptionType. |
| children | excep-category wind |
| used by | element; scribe-windType/exception. |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. |

scribe-wind-listType

| diagram |  |
| --- | --- |
| children | wind |
| used by | element; scribe-parametersType/wind-list. |
| attributes | Name; Type; Use; Default; Fixed. units; scribe-wind-unitsType; required. |

scribe-wind-speedType

| diagram |  |
| --- | --- |
| children | lower-limit upper-limit |
| used by | element; scribe-basic-windType/wind-speed. |

scribe-windType

| diagram |  |
| --- | --- |
| type | extension of scribe-basic-windType |
| properties | base; scribe-basic-windType. |
| children | wind-speed gust-speed exception |
| attributes | Name; Type; Use; Default; Fixed. start; xsd:dateTime; required. end; xsd:dateTime; required. direction; scribe-wind-directionType; required. modificator; scribe-wind-modificatorsType; optional. |

shadowingType

| diagram |  |
| --- | --- |
| children | shadowing-starts shadowing-ends |
| used by | element; instrumentType/shadowing. |
| attributes | Name; Type; Use; Default; Fixed. start-date; xsd:gMonthDay; required. end-date; xsd:gMonthDay; required. |

snapshot-cameraInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/snapshot-camera-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; snapshot-cameraTypeType; required. |

snapshot-cameraObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/snapshot-camera. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; snapshot-cameraTypeType; required. |

snowInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/snow-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; snowTypeType; required. |

snowObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/snow. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; snowTypeType; required. |

snowType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/snow. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; snowTypeType; required. height; xsd:nonNegativeInteger; optional. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

sourceType

| diagram |  |
| --- | --- |
| children | more-information production-center disclaimer credit credit-logo feedback |
| used by | element; headType/source. |

start-valid-timeType

| diagram |  |
| --- | --- |
| type | extension of xsd:dateTime |
| properties | base; xsd:dateTime. |
| used by | element; time-layoutElementType/start-valid-time. |
| attributes | Name; Type; Use; Default; Fixed. period-name; xsd:string; optional. |

station-metadataType

| diagram |  |
| --- | --- |
| type | extension of base-metadataType |
| properties | base; base-metadataType. |
| children | name description city country province state climatic-zone district timezone data-owner point station-state category type installation-date commission-date last-maintenance-date last-inspection-date station-owner station-maintainer rpu reporting-frequency vegetation pressure-instrument wind-instrument temperature-instrument precipitation-instrument radiation-instrument visibility-instrument pavement-instrument subsurface-instrument air-quality-instrument humidity-instrument snow-instrument weather-instrument snapshot-camera-instrument extension-instrument |
| used by | element; location-metadataType/station-metadata. |

subsurfaceInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing sensor-depth |
| used by | element; station-metadataType/subsurface-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; subsurfaceTypeType; required. |

subsurfaceObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/subsurface. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; subsurfaceTypeType; required. |

subsurfaceType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/subsurface. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. lane-number; xsd:nonNegativeInteger; optional; 1. type; subsurfaceTypeType; required. sensor-depth; xsd:nonNegativeInteger; optional; 5. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

supplementary-metadataType

| diagram |  |
| --- | --- |
| children | data-owner conversion-definitions categorical-definitions sampling-definitions |
| used by | element; metadataType/supplementary-metadata. |

temperatureInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/temperature-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; temperatureTypeType; required. |

temperatureObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/temperature. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; temperatureTypeType; required. |

temperatureType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/temperature. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; temperatureTypeType; required. units; unitType; required. categorical-code; xsd:string; optional. height; xsd:nonNegativeInteger; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

time-layoutElementType

| diagram |  |
| --- | --- |
| children | layout-key start-valid-time end-valid-time |
| used by | element; pavement-forecastType/time-layout. |
| attributes | Name; Type; Use; Default; Fixed. time-coordinate; time-coordinateType; required. summarization; summarizationType; required. |

timezoneType

| diagram |  |
| --- | --- |
| type | extension of timezonetypeType |
| properties | base; timezonetypeType. |
| used by | element; base-metadataType/timezone. |
| attributes | Name; Type; Use; Default; Fixed. offset; offsetType; required. |

valueCategoryType

| diagram |  |
| --- | --- |
| type | extension of xsd:string |
| properties | base; xsd:string. |
| used by | element; categorical-tableType/value. |
| attributes | Name; Type; Use; Default; Fixed. category; non-emptyString; required. |

valueType

| diagram |  |
| --- | --- |
| properties | mixed; true. |
| used by | element; measurementType/value. |
| attributes | Name; Type; Use; Default; Fixed. units; unitType; required. |

vegetationType

| diagram |  |
| --- | --- |
| children | type age height proximity direction |
| used by | element; station-metadataType/vegetation. |

visibilityInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/visibility-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; visibilityTypeType; required. |

visibilityObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/visibility. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; visibilityTypeType; required. |

visibilityType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/visibility. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; visibilityTypeType; required. height; xsd:nonNegativeInteger; optional. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

weatherInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/weather-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; weatherTypeType; required. |

weatherObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/weather. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; weatherTypeType; required. |

weatherType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/weather. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; weatherTypeType; required. height; xsd:nonNegativeInteger; optional. units; unitType; required. categorical-code; xsd:string; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

windInsType

| diagram |  |
| --- | --- |
| type | extension of instrumentType |
| properties | base; instrumentType. |
| children | description instrument-state units accuracy resolution sampling-interval lower-range upper-range instrument-id point equipment-information installation-date commission-date last-maintenance-date last-inspection-date calibration-history lane-number road-construction shadowing |
| used by | element; station-metadataType/wind-instrument. |
| attributes | Name; Type; Use; Default; Fixed. cmml-index; xsd:nonNegativeInteger; required. type; windTypeType; required. |

windObsType

| diagram |  |
| --- | --- |
| type | extension of measurementType |
| properties | base; measurementType. |
| children | qualifier value qc |
| used by | element; observationType/wind. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; windTypeType; required. |

windType

| diagram |  |
| --- | --- |
| children | name value |
| used by | element; parametersType/wind. |
| attributes | Name; Type; Use; Default; Fixed. index; xsd:nonNegativeInteger; required. type; windTypeType; required. units; unitType; required. categorical-code; xsd:string; optional. height; xsd:nonNegativeInteger; optional. time-layout; time-layoutAttributeType; required. categorical-table; xsd:string; optional. conversion-table; xsd:string; optional. sampling-table; xsd:string; optional. |

Simple Types

air-qualityTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; air-qualityType/@type air-qualityObsType/@type air-qualityInsType/@type. |

bridge-or-overpass-materialType

| type | restriction of xsd:string |
| --- | --- |
| used by | elements; bridgeType/material overpassType/material. |

calibrationCommentType

| type | xsd:string |
| --- | --- |
| used by | element; calibrationType/comment. |

CA-province-codeType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; base-metadataType/province. |

categoryType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; productType/category. |

datumType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; elevationType/@datum. |

direction-codeType

| type | restriction of xsd:string |
| --- | --- |
| used by | elements; roadwayType/aspect vegetationType/direction. |

earthworkType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; roadType/earthwork. |

elevation-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; elevationType/@units. |

emailType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; contactType/email. |

fieldType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; productType/field. |

humidityTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; humidityType/@type humidityObsType/@type humidityInsType/@type. |

identifiertypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; identifierType/@type. |

instrumentStateType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; basic-instrumentType/instrument-state. |

ip-addressType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; rpuType/ip-address. |

ISO-country-codeType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; base-metadataType/country. |

layout-keyType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; layout-key. |

measurement-categoryType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; qc-flagType/@associated-measurement-category. |

measurement-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; qc-flagType/@associated-measurement-type. |

non-emptyString

| type | restriction of xsd:string |
| --- | --- |
| used by | element; contactType/name. attributes; parametersType/@applicable-location valueCategoryType/@category qcType/@performer cmmlType/@version. |

offsetType

| type | restriction of xsd:decimal |
| --- | --- |
| used by | attribute; timezoneType/@offset. |

operational-modeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; productType/@operational-mode. |

origintypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; originType/@type. |

pavementTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; pavementType/@type pavementObsType/@type pavementInsType/@type. |

phone-numberType

| type | restriction of xsd:string |
| --- | --- |
| used by | elements; rpuType/phone-number contactType/phone-number. |

physical-statusTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; physical-statusType/@type physical-statusObsType/@type physical-statusInsType/@type. |

precipitationTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; precipitationType/@type precipitationObsType/@type precipitationInsType/@type. |

pressureTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; pressureType/@type pressureObsType/@type pressureInsType/@type. |

provinceType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; cityType/@province nws-zoneType/@province. |

qc-flagtypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; qc-flagType/@type. |

qc-flagvalueType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; qc-flagType/@value. |

radiationTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; radiationType/@type radiationObsType/@type radiationInsType/@type. |

radius-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; radiusType/@radius-units. |

scribe-accum-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-accum-amountType/@type. |

scribe-accum-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-accum-listType/@units. |

scribe-basic-simple-cloud-coverType

| type | restriction of xsd:nonNegativeInteger |
| --- | --- |
| used by | complexType; scribe-basic-cloud-coverType. |

scribe-cloud-cover-ceiling-codeType

| type | restriction of xsd:positiveInteger |
| --- | --- |
| used by | attribute; scribe-basic-cloud-coverType/@ceiling-code. |

scribe-cloud-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-cloud-listType/@units. |

scribe-excep-codeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-excep-categoryType/@excep-code. |

scribe-excep-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-excep-categoryType/@excep-type. |

scribe-freezing-spray-frequencyType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-freezing-sprayType/@frequency. |

scribe-freezing-spray-ice-cover-modifierType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-freezing-sprayType/@ice-cover-modifier. |

scribe-freezing-spray-intensityType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-freezing-sprayType/@intensity. |

scribe-freezing-spray-occurType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-freezing-sprayType/@occurrence. |

scribe-langType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; scribe-msc-zone-nameType/@lang scribe-excep-descriptionType/@lang. |

scribe-msc-zone-code-statusType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-msc-zone-codeType/@status. |

scribe-precipitation-frequencyType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-precipitation-eventType/@frequency. |

scribe-precipitation-intensityType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-precipitation-eventType/@intensity. |

scribe-precipitation-occurType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-precipitation-eventType/@occurrence. |

scribe-precipitation-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-precipitation-eventType/@type. |

scribe-simpe-probabilityType

| type | restriction of xsd:nonNegativeInteger |
| --- | --- |
| used by | complexTypes; scribe-ice-coverType scribe-probability-of-precipitationType. |

scribe-snow-level-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-snow-level-listType/@units. |

scribe-statusType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; productType/status. |

scribe-temperature-ground-frostType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-temperature-valueType/@ground-frost. |

scribe-temperature-trendType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-temperature-valueType/@trend. |

scribe-temperature-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-temperature-listType/@type. |

scribe-temperature-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-temperature-listType/@units. |

scribe-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; productType/type. |

scribe-value-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-valueType/@units. |

scribe-visibility-frequencyType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-visibilityType/@frequency. |

scribe-visibility-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-visibilityType/@type. |

scribe-warning-codeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-warning-eventType/@code. |

scribe-warning-statusType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-warning-eventType/@status. |

scribe-warning-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-warning-eventType/@type. |

scribe-wave-height-ice-cover-modifierType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-wave-heightType/@ice-cover-modifier. |

scribe-wave-height-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-wave-height-listType/@units. |

scribe-wind-directionType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-windType/@direction. |

scribe-wind-modificatorsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-basic-windType/@modificator. |

scribe-wind-unitsType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; scribe-wind-listType/@units. |

slopeType

| type | xsd:decimal |
| --- | --- |
| used by | element; roadwayType/slope. |

snapshot-cameraTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; snapshot-cameraObsType/@type snapshot-cameraInsType/@type. |

snowTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; snowType/@type snowObsType/@type snowInsType/@type. |

stateType

| type | restriction of xsd:string |
| --- | --- |

station-categoryType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; station-metadataType/category. |

stationStateType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; station-metadataType/station-state. |

station-typeType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; station-metadataType/type. |

subsurfaceTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; subsurfaceType/@type subsurfaceObsType/@type subsurfaceInsType/@type. |

summarizationType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; time-layoutElementType/@summarization cityType/@summarization nws-zoneType/@summarization. |

summaryType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; qcType/summary. |

surfaceType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; roadwayType/surface. |

temperatureTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; temperatureType/@type temperatureObsType/@type temperatureInsType/@type. |

time-coordinateType

| type | restriction of xsd:string |
| --- | --- |
| used by | attribute; time-layoutElementType/@time-coordinate. |

time-layoutAttributeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; pressureType/@time-layout windType/@time-layout temperatureType/@time-layout precipitationType/@time-layout radiationType/@time-layout visibilityType/@time-layout pavementType/@time-layout subsurfaceType/@time-layout snowType/@time-layout humidityType/@time-layout air-qualityType/@time-layout weatherType/@time-layout physical-statusType/@time-layout extensionType/@time-layout. |

timeUnitsType

| type | restriction of xsd:string |
| --- | --- |

timezonetypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | complexType; timezoneType. |

unitType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; basic-instrumentType/units. attributes; valueType/@units qualifierType/@units sampling-tableType/sampling-interval/@units conversion-tableType/@units categorical-tableType/@units extensionType/@units physical-statusType/@units weatherType/@units air-qualityType/@units humidityType/@units snowType/@units subsurfaceType/@units pavementType/@units visibilityType/@units radiationType/@units precipitationType/@units temperatureType/@units windType/@units pressureType/@units. |

US-state-codeType

| type | restriction of xsd:string |
| --- | --- |
| used by | element; base-metadataType/state. |

visibilityTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; visibilityType/@type visibilityObsType/@type visibilityInsType/@type. |

weatherTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; weatherType/@type weatherObsType/@type weatherInsType/@type. |

windTypeType

| type | restriction of xsd:string |
| --- | --- |
| used by | attributes; windType/@type windObsType/@type windInsType/@type. |

- Valid Head Types

| category types |  |  |  |
| --- | --- | --- | --- |
| forecast | analysis | observation | statistics |
| metadata |  |  |  |

Table D-1: Valid types for category

| field types |  |
| --- | --- |
| meteorological | hydrological |
| oceanographical | land surface |
| space |  |

Table D-2: Valid types for field

| operational-mode types |  |  |  |
| --- | --- | --- | --- |
| official | developmental | experimental | test |

Table D-3: Valid types for operational-mode

| product status values |  |  |
| --- | --- | --- |
| English | French | Description |
| active | actif | Product is issued for the current season. |
| inactive season | saison inactive | Product is not issued for the current season. |
| temporarily inactive | inactif temporairement | Product is temporarily not issued. |

Table D-4:  Valid types for product status

| product type values |  |  |
| --- | --- | --- |
| English | French | Description |
| regular | régulier | Product is issued on normal issue time. |
| modification | modification | Product is issued out of the regular issue time to indicate a modification from the regular issue (revised forecast). |

Table D-5: Valid types for product type

- Valid Forecast Types

Note: Double exclamation (“!!”) marked types are supported by NTCIP 1204 v02.17

| pavement-forecast air-quality types |  |  |  |
| --- | --- | --- | --- |
| carbon-monoxide!! | carbon-dioxide!! | nitrous-oxide!! | nitrogen-dioxide!! |
| sulfur-dioxide!! | ozone!! | small-particulate!! |  |

Table E-6: Valid types for pavement-forecast air-quality

| pavement-forecast datum types |  |  |
| --- | --- | --- |
| station-reference-level | mean-sea-level | pavement-surface |

Table E-7: Valid pavement-forecast datum types

| pavement-forecast elevation-units types |  |  |
| --- | --- | --- |
| cm | in | ft |
| m |  |  |

Table E-8: Valid pavement-forecast elevation-units types

| pavement-forecast humidity types |  |  |
| --- | --- | --- |
| relative-humidity!! | average-relative-humidity | maximum-relative-humidity |
| relative-humidity-at-minimum-temperature | relative-humidity-at-maximum-temperature |  |

Table E-9: Valid types for pavement-forecast humidity

| pavement-forecast identifiertype types |  |  |
| --- | --- | --- |
| client | network |  |

Table E-10: Valid pavement-forecast identifiertype types

| pavement-forecast origintype types |  |  |
| --- | --- | --- |
| station | region |  |

Table E-11: Valid pavement-forecast origintype types

| pavement-forecast pavement types |  |  |  |
| --- | --- | --- | --- |
| sensor-location!! | type!! | elevation!! | solar-energy!! |
| sensor-type!! | temperature!! | temperature-at-2-to-10-cm-depth!! | salinity!! |
| freeze-point!! | black-ice!! | sensor-error!! | water-depth!! |
| conductivity!! | ice-thickness!! | average-temperature | average-conductivity |
| conductivity-code | average-conductivity-compensated | conductivity-code-compensated | surface-state |
| error-status | surface-status!! | water-or-ice-thickness |  |

Table E-12: Valid types for pavement-forecast pavement

| pavement-forecast physical-status types |  |  |
| --- | --- | --- |
| door-open!! | battery-minimum-voltage | battery-maximum-voltage |

Table E-13: Valid types for pavement-forecast physical-status

| pavement-forecast precipitation types |  |  |  |
| --- | --- | --- | --- |
| water-depth!! | moisture-on-sensor!! | rate!! | situation!! |
| start-time!! | end-time!! | total-over-10minutes | total-over-hour!! |
| total-over-3hours!! | total-over-6hours!! | total-over-12hours!! | total-over-24hours!! |
| situation-WMO4680 | situation-WMO4680-over-15minutes | situation-WMO4680-over-hour | temporal-dispersion |
| coverage | probability | rainfall | qualifier |

Table E-14: Valid types for pavement-forecast precipitation

| pavement-forecast pressure types |  |
| --- | --- |
| atmospheric!! | average-atmospheric |

Table E-15: Valid types for pavement-forecast pressure

| pavement-forecast radiation types |  |  |  |
| --- | --- | --- | --- |
| total-sun!! | cloud-situation!! | instantaneous-terrestrial-radiation!! | instantaneous-solar-radiation!! |
| total-radiation!! | total-radiation-period!! | average-energy-long-wave-up | average-energy-short-wave-down |
| average-energy-long-wave-down | average-energy-short-wave-up | average-albedo | average-net-solar |
| average-net-infrared | average-sky-temperature | average-ground-temperature | average-net-total-radiation |

Table E-16: Valid types for pavement-forecast radiation

| pavement-forecast snow types |  |  |  |
| --- | --- | --- | --- |
| adjacent-snow-depth!! | roadway-unpacked-snow-depth!! | roadway-packed-snow-depth!! | snowfall-accumulation-rate!! |
| roadway-snow-depth | roadway-maximum-snow-depth | roadway-minimum-snow-depth | distance-between-sensor-and-snow |
| maximum-distance-between-sensor-and-snow | minimum-distance-between-sensor-and-snow | snowfall |  |

Table E-17: Valid types for pavement-forecast snow

| pavement-forecast subsurface types |  |  |  |
| --- | --- | --- | --- |
| sensor-location!! | type!! | temperature!! | moisture!! |
| sensor-error!! | average-temperature | sensor-depth!! |  |

Table E-18: Valid types for pavement-forecast subsurface

| pavement-forecast summarization types |  |  |
| --- | --- | --- |
| none | mean | medium |
| mode | maximum | minimum |

Table E-19: Valid pavement-forecast summarization types

| pavement-forecast temperature types |  |  |  |
| --- | --- | --- | --- |
| air-temperature!! | wet-bulb!! | dew-point!! | maximum-air-temperature-over-24hours!! |
| minimum-air-temperature-over-24hours!! | average-air-temperature | average-dew-point | minimum-air-temperature |
| dew-point-at-minimum-temperature | maximum-air-temperature | dew-point-at-maximum-temperature | air-temperature-at-maximum-relative-humidity |
| dew-point-at-maximum-relative-humidity |  |  |  |

Table E-20: Valid types for pavement-forecast temperature

| pavement-forecast time-coordinate types |  |  |
| --- | --- | --- |
| UTC | local |  |

Table E-21: Valid pavement-forecast time-coordinate types

| pavement-forecast unit types |  |  |  |
| --- | --- | --- | --- |
| km/h | unitless | W/m2 | kg/kg |
| mm/h | mS | % | V |
| bitmap | m/s | degF | degC |
| K | deg | mm | cm |
| m | km | in | ft |
| statute-miles | ppm | ppb | h |
| min | s | mb | string |
| code | micrograms/m3 | milli-mhos/10cm | image |

Table E-22: Valid pavement-forecast unit types

| pavement-forecast visibility types |  |
| --- | --- |
| distance!! | situation!! |

Table E-23: Valid types for pavement-forecast visibility

| pavement-forecast wind types |  |  |  |
| --- | --- | --- | --- |
| average-direction!! | direction!! | speed!! | situation!! |
| maximum-speed!! | direction-at-maximum-speed!! | average-scalar-speed-last-2minutes | average-vectorial-speed-last-2minutes |
| average-direction-last-2minutes | standard-deviation-of-direction-last-2minutes | average-scalar-speed-over-10minutes | average-vectorial-speed-over-10minutes |
| average-direction-over-10minutes | standard-deviation-of-direction-over-10minutes | standard-deviation-of-direction-over-60minutes | average-scalar-speed-over-60minutes |
| average-vectorial-speed-over-60minutes | average-speed!! |  |  |

Table E-24: Valid types for pavement-forecast wind

| pavement-forecast weather types |  |  |
| --- | --- | --- |
| condition | alert | cloud-coverage |

Table E-25: Valid types for pavement-forecast weather

| meteocode-forecast accumulation types |  |  |
| --- | --- | --- |
| English | French | Description |
| ice pellet | grésil | Ice pellet amount in cm |
| snow | neige | Snow amount in cm |
| rain | pluie | Rain amount in mm |
| freezing rain | pluie verglaçante | Freezing amount in mm |

Table E-26: Valid values for meteocode-forecast accumulation types

| meteocode-forecast exception codes |  |  |
| --- | --- | --- |
| English | French | Description |
| area | zone |  |
| period | période |  |
| region | région |  |
| sector | secteur |  |
| topo | topo | Topographic feature |
| city | ville |  |

Table E-27: Valid values for meteocode-forecast exception codes

| meteocode-forecast exception types |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| except | sauf | Weather element value for a specific location that differs from the main conditions. |
| only | seulement | Weather element that applies only to a specific location. |
| mainly | surtout | N/A |

Table E-28: Valid values for meteocode-forecast exception types

| meteocode-forecast freezing spray frequency |  |  |
| --- | --- | --- |
| English | French | Description |
| continuous | continu | Freezing spray present continously |
| occasional | occasionnel | Freezing spray present at times |

Table E-29: Valid types for meteocode-forecast freezing spray frequency

| meteocode-forecast freezing spray intensity |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| heavy | fort |  |
| moderate | modéré |  |
| light | faible |  |

Table E-30: Valid types for meteocode-forecast freezing spray intensity

| meteocode-forecast freezing spray occurrence |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| certain | certain |  |
| possible | possible |  |
| risk | risque | Risk: less than 50% chance of occurence |

Table E-31: Valid types for meteocode-forecast freezing spray occurrence

| meteocode-forecast freezing spray ice cover modifier |  |  |
| --- | --- | --- |
| English | French | Description |
| open water | en eaux libres |  |
| outside ice edge | à l’extérieur de la limite des glaces |  |
| ice covered | couvert de glace |  |

Table E-32: Valid types for meteocode-forecast freezing spray ice cover modifier

| meteocode-forecast msc zone code status |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| active | actif | Region forecast issued for the current season |
| active last season | actif dernière saison | Last region forecast issued for the current season |
| active last temporarily | actif dernier temporairement | Last region forecast issued before a temporary closing period |
| inactive season | saison inactive | Region forecast not issued for the current season |
| temporarily inactive | inactif temporairement | Region forecast not issued for a temporary period |

Table E-33: Valid values for meteocode-forecast msc zone code status

| meteocode-forecast precipitation frequency |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| brief | bref | Precipitations that are not continuous |
| few | peu | Precipitations that are not continuous |
| frequent | frequent | Precipitations that are not continuous |
| occasional | occasionnel | Precipitations that are not continuous |
| continuous | continu | Precipitations that fall continuously |

Table E-34: Valid types for meteocode-forecast precipitation frequency

| meteocode-forecast precipitation intensity |  |  |
| --- | --- | --- |
| English | French | Description |
| heavy | fort |  |
| moderate | modéré |  |
| nil | nil |  |
| light | faible |  |
| very light | très faible | Future development. |

Table E-35: Valid types for meteocode-forecast precipitation intensity

| meteocode-forecast precipitation occurrence |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| certain | certain |  |
| possible | possible |  |
| risk | risque | Risk: less than 50% chance of occurence |

Table E-36: Valid types for meteocode-forecast precipitation occurrence

| meteocode-forecast precipitation types |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| hail | grêle |  |
| blizzard | blizzard |  |
| blowing snow | poudrerie |  |
| waterspout | trombe marine |  |
| ice cristal | cristaux de glace |  |
| ice pellet | grésil |  |
| drizzle | bruine |  |
| rain | pluie |  |
| shower | averses |  |
| snow | neige |  |
| wet snow | neige fondante |  |
| snow squall | bourrasque de neige |  |
| flurries | averses de neige |  |
| wet snow flurries | averses de neige fondante |  |
| thunderstorm | orages |  |
| freezing drizzle | bruine verglaçante |  |
| freezing rain | pluie verglaçante |  |

Table E-37: Valid types for meteocode-forecast precipitation types

| meteocode-forecast temperature types |  |  |
| --- | --- | --- |
| English | French | Description |
| air | température de l’air | Surface air temperature |
| dew-point | point de rosée | Dew-point temperature |
| climatology | climatologie | Climatological air temperature |
| sea-surface | température à la surface de la mer | Sea surface temperature |

Table E-38: Valid values for meteocode-forecast temperature type

| meteocode-forecast temperature ground-frost |  |  |
| --- | --- | --- |
| English | French | Description1 |
| frost | gel |  |
| widespread frost | gel généralisé |  |
| ground frost | gel au sol |  |
| risk of ground frost | risque de gel au sol |  |

Table E-39:  Valid values for meteocode-forecast temperature ground-frost

| meteocode-forecast temperature trend |  |  |
| --- | --- | --- |
| English | French | Description |
| fall | baisse |  |
| rise | hausse |  |
| interpolated | interpolé | Interpolated temperature values |
| max | max |  |
| min | min |  |
| intermediate point | point intermédiaire | Additional temperature values used to describe the evolution of changes. |
| stationary | stationnaire |  |

Table E-40: Valid types for meteocode-forecast temperature trend

| meteocode-forecast wave height ice cover modifier |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| open water | en eaux libres |  |
| outside ice edge | au-delà de la limite des glaces |  |
| ice covered | couvert de glace |  |

Table E-41: Valid types for meteocode-forecast wave-height ice cover modifier

| meteocode-forecast visibility types |  |  |
| --- | --- | --- |
| English | French | Description |
| fog banks | bancs de brouillard |  |
| ice fog banks | bancs de brouillard glacé |  |
| fog | brouillard |  |
| areas of fog | brouillard par endroits |  |
| ice fog | brouillard glace |  |
| mist | brume |  |
| areas of mist | brume par endroits |  |
| haze | brume sèche |  |
| foggy | brumeux |  |
| fog patches | brumeux par endroits |  |
| blowing dust | chasse-poussière |  |
| smoke | fumée |  |
| sea smoke | fumée de mer |  |
| fog patches | nappes de brouillard |  |
| ice fog patches | nappes de brouillard glacé |  |
| precipitation | précipitations | Due to precipitation |
| blowing snow | poudrerie |  |
| drifting snow | poudrerie basse |  |
| local drifting snow | poudrerie par endroits |  |
| local blowing snow | poudrerie haute par endroits |  |
| smog | smog |  |

Table E-42: Valid types for meteocode-forecast visibility

| meteocode-forecast visibility frequency |  |  |
| --- | --- | --- |
| English | French | Description |
| continuous | continu | Spacial or temporal coverage near 100% |
| occasional | occasionnel | Spacial or temporal coverage 26% to 50% |

Table E-43: Valid types for meteocode-forecast visibility frequency

| meteocode-forecast warning types |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| warning | avertissement |  |
| advisory | avis |  |

Table E-44: Valid types for meteocode-forecast warning type

| meteocode-forecast warning code |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| arctic air | air arctique | Arctic outflow |
| blizzard | blizzard |  |
| snow squall | bourrasques de neige |  |
| freezing drizzle | bruine verglaçante |  |
| blowind dust | chasse-poussière |  |
| gale | coup de vent |  |
| others | divers | Weather warning |
| freezing spray | embrun verglaçant |  |
| widespread frost | gel generalisé |  |
| ground frost | gel au sol |  |
| hail | grêle | Hail storm |
| ice pellet | grésil |  |
| humidex | humidex |  |
| squall line | ligne de grain |  |
| snow | neige |  |
| heavy snow | neige abondante |  |
| heavy snow and blowing snow | neige abondante et poudrerie |  |
| snow and ice pellet | neige et grésil |  |
| snow and blowing snow | neige et poudrerie |  |
| nil | nil |  |
| storm surge | onde de tempête |  |
| marine thunderstorm | orage marin |  |
| severe thunderstorm | orage violent |  |
| hurricane | ouragan |  |
| heavy rain | pluie abondante |  |
| rain and freezing drizzle | pluie et bruine verglaçante |  |
| freezing rain | pluie verglaçante |  |
| blowing snow | poudrerie |  |
| air quality | qualité de l’air |  |
| flash freeze | refroidissement soudain |  |
| wind chill | refroidissement éolien |  |
| winter storm | tempête hivernale |  |
| tropical storm | tempête tropicale |  |
| tornado | tornade |  |
| waterspout | trombe marine |  |
| heat wave | vague de chaleur |  |
| cold wave | vague de froid |  |
| marine winds | vents marins |  |
| suêtes winds | vents suêtes |  |
| storm winds | vents de tempête |  |
| strong winds | vents violents |  |
| wreckhouse winds | vents wreckhouse |  |

Table E-45: Valid values for meteocode-forecast warning codes

| meteocode-forecast warning status |  |  |
| --- | --- | --- |
| English | French | Description |
| cancelled | annulé |  |
| downgraded | abaissé |  |
| issued | émis |  |
| in effect | en vigueur |  |
| end | fin |  |
| upgraded | haussé |  |
| maintained | maintenu |  |
| updated | mis à jour |  |
| nil | nil |  |

Table E-46: Valid values for meteocode-forecast warning status

| meteocode-forecast wind direction |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| east | est |  |
| nil | nil |  |
| north | nord |  |
| northeast | nord-est |  |
| northwest | nord-ouest |  |
| west | ouest |  |
| easterly | du secteur est |  |
| northerly | du secteur nord |  |
| northeasterly | du secteur nord-est |  |
| northwesterly | du secteur nord-ouest |  |
| westerly | du secteur ouest |  |
| southerly | du secteur sud |  |
| southeasterly | du secteur sud-est |  |
| southwesterly | du secteur sud-ouest |  |
| south | sud |  |
| southeast | sud-est |  |
| southwest | sud-ouest |  |
| variable | variable | From any direction |

Table E-47: Valid values for meteocode-forecast wind direction

| meteocode-forecast wind modificators |  |  |
| --- | --- | --- |
| English | French | Description |
| onshore | vers la côte |  |
| offshore | de la côte |  |
| inflow | de la mer |  |
| outflow | de la terre |  |

Table E-48: Valid values for meteocode-forecast wind modificators

| meteocode-forecast accum-units types |  |  |
| --- | --- | --- |
| English | French | Description |
| cm | cm | Centimeter |
| hndin | hndin | Hundred of inches |
| in | in | Inch |
| mm | mm | Millimeters |

Table E-49:  Valid unit types for meteocode-forecast accumulation

| meteocode-forecast cloud-units types |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| deci | deci | Tenth |
| octo | octo | Eighth |
| percent | pourcent | Percent |

Table E-50:  Valid unit types for meteocode-forecast cloud

| meteocode-forecast snow-level-units types |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| m | m | Meter |

Table E-51:  Valid unit types for meteocode-forecast snow level

| meteocode-forecast temperature-unit types |  |  |
| --- | --- | --- |
| English | French | Description |
| celsius | celsius | Celsius |
| fahrenheit | fahrenheit | Fahrenheit |

Table E-52:  Valid unit types for meteocode-forecast temperature

| meteocode-forecast value-units types |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| m | m | Meter |
| mi | mi | Mile |
| km | km | Kilometer |
| NM | MN | Nautical mile |

Table E-53:  Valid unit types for meteocode-forecast visibility value

| meteocode-forecast wave-height-units types |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| m | m | Meter |
| ft | ft | Feet |

Table E-54:  Valid unit types for meteocode-forecast wave height

| meteocode-forecast wind-units types |  |  |
| --- | --- | --- |
| English | French1 | Description2 |
| kmh | kmh | Kilometer per hour |
| kt | nd | Knot |
| msec | msec | Meter per second |
| mph | mph | Mile per hour |

Table E-55:  Valid unit types for meteocode-forecast wind

| Meteocode-forecast cloud-cover-ceiling-code values |  |  |
| --- | --- | --- |
| English | French | Description (All heights are above ground) |
| 1 | 1 | Height from 0 to 199 feet. |
| 2 | 2 | Height from 200 to 499 feet. |
| 3 | 3 | Height from 500 to 999 feet. |
| 4 | 4 | Height from 1000 to 2999 feet. |
| 5 | 5 | Height from 3000 to 4999 feet. |
| 6 | 6 | Height from 5000 to 7999 feet. |
| 7 | 7 | Height from 8000 to 11999 feet. |
| 8 | 8 | Height from 12000 to 19999 feet. |
| 9 | 9 | Height of and above 20000 feet. |

Table E-56:  Valid values for meteocode-forecast cloud cover ceiling-code

- Valid Observation Types

air-quality types: refer to Table E-6 in Appendix E

datum types: refer to Table E-7 in Appendix E

elevation-units types: refer to Table E-8 in Appendix E

humidity types: refer to Table E-9 in Appendix E

identifiertype types: refer to Table E-10 in Appendix E

origintype types: refer to Table E-11 in Appendix E

pavement types: refer to Table E-12 in Appendix E

physical-status types: refer to Table E-13 in Appendix E

precipitation types: refer to Table E-14in Appendix E

pressure types: refer to Table E-15 in Appendix E

radiation types: refer to Table E-16 in Appendix E

snow types: refer to Table E-17 in Appendix E

subsurface types: refer to Table E-18 in Appendix E

temperature types: refer to Table E-20 in Appendix E

unit types: refer to Table E-22 in Appendix E

visibility types: refer to Table E-23 in Appendix E

weather types: refer to Table E-25 in Appendix E

wind types: refer to Table E-24in Appendix E

| measurement-category types |  |  |  |
| --- | --- | --- | --- |
| pressure | wind | temperature | precipitation |
| radiation | visibility | pavement | subsurface |
| air-quality | physical-status | humidity | weather |
| snow | snapshot-camera |  |  |

Table F-57: Valid types for measurement-category

| qc-flagtype types |  |  |  |
| --- | --- | --- | --- |
| presence | integrity | range | temporal |
| inter-variable | spatial |  |  |

Table F-58: Valid types for qc-flagtype

| qc-flagvalue types |  |  |  |
| --- | --- | --- | --- |
| missing | error | doubtful | inconsistency |
| estimated | corrected |  |  |

Table F-59: Valid types for qc-flagvalue

| qualifier types |  |  |  |
| --- | --- | --- | --- |
| conversion-table | lane-number | sampling-table | categorical-table |
| height | name |  |  |

Table F-60: Valid types for qualifier

| snapshot-camera types |  |  |
| --- | --- | --- |
| camera-location!! | camera-error!! | filename!! |
| image!! |  |  |

Table F-61: Valid types for snapshot-camera

| summary types |  |  |  |
| --- | --- | --- | --- |
| accepted/passed | error | missing | doubtful |
| inconsistency | corrected | estimated |  |

Table F-62: Valid types for (QC) summary

| timezone types |  |  |  |
| --- | --- | --- | --- |
| ADT | AST | CDT | CST |
| EDT | EST | MDT | MST |
| NDT | NST | PDT | PST |
| AKST | AKDT | HAST | HADT |
| SST |  |  |  |

Table F-63: Valid types for timezone

- Valid Metadata Types

Air-quality-sensor types: refer to Table E-6 in Appendix E .

| Bridge or Overpass Material |  |  |  |
| --- | --- | --- | --- |
| steel | PCC(solid) | PCC(hollow) | other |

Table G-64 Valid bridge-or-overpass-material values.

| Earthwork |  |  |
| --- | --- | --- |
| cut | fill | neutral |

Table G-65 Valid earthwork values.

Humidity-sensor-types: refer to Table E-9 in Appendix E .

| Instrument state types |  |  |
| --- | --- | --- |
| active-operational | active-not-reporting | active-qc-failure |
| active-element-missing | inactive-uninstalled | inactive-retired |
| inactive-suppressed | inactive-scheduled-maintenance | inactive-unscheduled-maintenance |

Table G-66 Valid instrument state qualifier values.

Pavement-sensor types: refer to Table E-12 in Appendix E .

Physical-status-sensor types: refer to Table E-13 in Appendix E .

Precipitation-sensor types: refer to Table E-14 in Appendix E .

Pressure-sensor types: refer to Table E-15 in Appendix E .

Radiation-sensor types: refer to Table E-16 in Appendix E .

Snapshot-camera-sensor types: refer to Table F-61 in .

Snow-sensor types: refer to Table E-17 in Appendix E .

| States |  |
| --- | --- |
| on | off |

Table G-67 Valid state values.

| Station Categories |  |  |
| --- | --- | --- |
| permanent | mobile | transportable |

Table G-68 Valid station-category values.

| Station state types |  |  |
| --- | --- | --- |
| active-operational | active-testing | active-suppressed |
| active-not-reporting | inactive-uninstalled | inactive-retired |
| inactive-suppressed | inactive-scheduled-maintenance | inactive-unscheduled-maintenance |

Table G-69 Valid station state qualifier values.

| Station Types |  |  |
| --- | --- | --- |
| automatic | staffed | unknown |

Table G-70 Valid station-type values.

Subsurface-sensor types: refer to Table E-18 in Appendix E .

| Surface types |  |  |  |
| --- | --- | --- | --- |
| PCC | asphalt | asphalt over PCC | other |

Table G-71 Valid surface values.

Temperature-sensor types: refer to Table E-20 in Appendix E .

| Time Units |  |  |
| --- | --- | --- |
| h | Min | s |

Table G-72 Valid timeUnits values.

Units: refer to Table E-22 in Appendix E .

Visibility-sensor types: refer to Table E-23 in Appendix E .

Weather-sensor types: refer to Table E-25 in Appendix E .

Wind-sensor types: refer to Table E-24 in Appendix E .

- Schema Organization

XSD File Structure

The indentation shows the relationship between xsd files. Example: The parameters.xsd schema file has a reference which enables it to use any of the type definitions defined in the attribute_list.xsd schema file.

cmml.xsd (top level)

- head.xsd

- data.xsd

  - forecast.xsd

    - scribe-forecast.xsd

      - scribe-location.xsd

      - scribe-parameters.xsd

        - scribe-prediction.xsd

          - scribe-attribute_list.xsd

    - time_layout.xsd

    - parameters.xsd

  - observation-series.xsd

    - observation.xsd

      - measurement.xsd

        - qc.xsd

  - metadata.xsd

    - location-metadata.xsd

      - instrument.xsd

        - attribute_list.xsd

        - location.xsd

          - origin.xsd

    - supplementary-metadata.xsd

Type Definition Location

| Complex Type | Definition Location |
| --- | --- |
| air-qualityInsType | instrument.xsd |
| air-qualityObsType | measurement.xsd |
| air-qualityType | parameters.xsd |
| areaType | location.xsd |
| base-metadataType | location-metadata.xsd |
| basic-instrumentType | instrument.xsd |
| bridgeType | instrument.xsd |
| calibration-historyType | instrument.xsd |
| calibrationType | instrument.xsd |
| categorical-tableType | supplementary-metadata.xsd |
| circleType | location.xsd |
| cityType | location.xsd |
| cmmlType | cmml.xsd |
| contactType | location-metadata.xsd |
| conversion-tableType | supplementary-metadata.xsd |
| creation-dateType | head.xsd |
| dataType | data.xsd |
| elevationType | location.xsd |
| equipment-informationType | instrument.xsd |
| extensionInsType | instrument.xsd |
| extensionType | parameters.xsd |
| forecast-locationType | location.xsd |
| forecastType | forecast.xsd |
| headType | head.xsd |
| humidityInsType | instrument.xsd |
| humidityObsType | measurement.xsd |
| humidityType | parameters.xsd |
| identifierType | origin.xsd |
| instrumentType | instrument.xsd |
| linestringType | location.xsd |
| location-metadataType | location-metadata.xsd |
| locationType | location.xsd |
| measurementType | measurement.xsd |
| messageType | qc.xsd |
| metadataType | metadata.xsd |
| nws-zoneType | location.xsd |
| observation-seriesType | observation-series.xsd |
| observationType | observation.xsd |
| originType | origin.xsd |
| overpassType | instrument.xsd |
| parametersType | parameters.xsd |
| pavement-forecastType | forecast.xsd |
| pavementInsType | instrument.xsd |
| pavementObsType | measurement.xsd |
| pavementType | parameters.xsd |
| physical-statusInsType | instrument.xsd |
| physical-statusObsType | measurement.xsd |
| physical-statusType | parameters.xsd |
| pointType | location.xsd |
| polygonType | location.xsd |
| precipitationInsType | instrument.xsd |
| precipitationObsType | measurement.xsd |
| precipitationType | parameters.xsd |
| pressureInsType | instrument.xsd |
| pressureObsType | measurement.xsd |
| pressureType | parameters.xsd |
| production-centerType | head.xsd |
| productType | head.xsd |
| qc-flagType | qc.xsd |
| qcType | qc.xsd |
| qualifierType | measurement.xsd |
| radiationInsType | instrument.xsd |
| radiationObsType | measurement.xsd |
| radiationType | parameters.xsd |
| radiusType | location.xsd |
| rectangleType | location.xsd |
| region-metadataType | location-metadata.xsd |
| road-constructionType | instrument.xsd |
| roadType | instrument.xsd |
| roadwayType | instrument.xsd |
| rpuType | location-metadata.xsd |
| sampling-tableType | supplementary-metadata.xsd |
| scribe-accum-amountType | scribe-prediction.xsd |
| scribe-accum-listType | scribe-parameters.xsd |
| scribe-accum-lower-limitType | scribe-prediction.xsd |
| scribe-accum-upper-limitType | scribe-prediction.xsd |
| scribe-amended-regionsType | head.xsd |
| scribe-basic-cloud-coverType | scribe-prediction.xsd |
| scribe-basic-exceptionType | scribe-prediction.xsd |
| scribe-basic-freezing-sprayType | scribe-prediction.xsd |
| scribe-basic-precipitation-eventType | scribe-prediction.xsd |
| scribe-basic-visibilityType | scribe-prediction.xsd |
| scribe-basic-windType | scribe-prediction.xsd |
| scribe-cloud-cover-exceptionType | scribe-prediction.xsd |
| scribe-cloud-coverType | scribe-prediction.xsd |
| scribe-cloud-listType | scribe-parameters.xsd |
| scribe-excep-categoryType | scribe-prediction.xsd |
| scribe-excep-descriptionType | scribe-prediction.xsd |
| scribe-forecastType | scribe-forecast.xsd |
| scribe-freezing-spray-exceptionType | scribe-prediction.xsd |
| scribe-freezing-spray-listType | scribe-parameters.xsd |
| scribe-freezing-sprayType | scribe-prediction.xsd |
| scribe-gust-speedType | scribe-prediction.xsd |
| scribe-ice-cover-listType | scribe-parameters.xsd |
| scribe-ice-coverType | scribe-prediction.xsd |
| scribe-locationType | scribe-location.xsd |
| scribe-msc-zone-codeType | scribe-location.xsd |
| scribe-msc-zone-nameType | scribe-location.xsd |
| scribe-parametersType | scribe-parameters.xsd |
| scribe-pointType | scribe-location.xsd |
| scribe-precipitation-event-exceptionType | scribe-prediction.xsd |
| scribe-precipitation-eventType | scribe-prediction.xsd |
| scribe-precipitation-listType | scribe-parameters.xsd |
| scribe-probability-of-precipitation-listType | scribe-parameters.xsd |
| scribe-probability-of-precipitationType | scribe-prediction.xsd |
| scribe-snow-level-listType | scribe-parameters.xsd |
| scribe-snow-levelType | scribe-prediction.xsd |
| scribe-temperature-listType | scribe-parameters.xsd |
| scribe-temperature-value-exceptionType | scribe-prediction.xsd |
| scribe-temperature-valueType | scribe-prediction.xsd |
| scribe-UV-index-listType | scribe-parameters.xsd |
| scribe-UV-indexType | scribe-prediction.xsd |
| scribe-valueType | scribe-prediction.xsd |
| scribe-visibility-exceptionType | scribe-prediction.xsd |
| scribe-visibility-listType | scribe-parameters.xsd |
| scribe-visibilityType | scribe-prediction.xsd |
| scribe-warning-eventType | scribe-prediction.xsd |
| scribe-warning-listType | scribe-parameters.xsd |
| scribe-wave-height-listType | scribe-parameters.xsd |
| scribe-wave-heightType | scribe-prediction.xsd |
| scribe-wind-exceptionType | scribe-prediction.xsd |
| scribe-wind-listType | scribe-parameters.xsd |
| scribe-wind-speedType | scribe-prediction.xsd |
| scribe-windType | scribe-prediction.xsd |
| shadowingType | instrument.xsd |
| snapshot-cameraInsType | instrument.xsd |
| snapshot-cameraObsType | measurement.xsd |
| snowInsType | instrument.xsd |
| snowObsType | measurement.xsd |
| snowType | parameters.xsd |
| sourceType | head.xsd |
| start-valid-timeType | time_layout.xsd |
| station-metadataType | location-metadata.xsd |
| subsurfaceInsType | instrument.xsd |
| subsurfaceObsType | measurement.xsd |
| subsurfaceType | parameters.xsd |
| supplementary-metadataType | supplementary-metadata.xsd |
| temperatureInsType | instrument.xsd |
| temperatureObsType | measurement.xsd |
| temperatureType | parameters.xsd |
| time-layoutElementType | time_layout.xsd |
| timezoneType | location-metadata.xsd |
| valueCategoryType | supplementary-metadata.xsd |
| valueType | measurement.xsd |
| vegetationType | location-metadata.xsd |
| visibilityInsType | instrument.xsd |
| visibilityObsType | measurement.xsd |
| visibilityType | parameters.xsd |
| weatherInsType | instrument.xsd |
| weatherObsType | measurement.xsd |
| weatherType | parameters.xsd |
| windInsType | instrument.xsd |
| windObsType | measurement.xsd |
| windType | parameters.xsd |

| Simple Type | Definition Location |
| --- | --- |
| air-qualityTypeType | attribute_list.xsd |
| bridge-or-overpass-materialType | instrument.xsd |
| calibrationCommentType | instrument.xsd |
| CA-province-codeType | attribute_list.xsd |
| categoryType | head.xsd |
| datumType | location.xsd |
| direction-codeType | attribute_list.xsd |
| earthworkType | instrument.xsd |
| elevation-unitsType | location.xsd |
| emailType | attribute_list.xsd |
| fieldType | head.xsd |
| humidityTypeType | attribute_list.xsd |
| identifiertypeType | origin.xsd |
| instrumentStateType | instrument.xsd |
| ip-addressType | attribute_list.xsd |
| ISO-country-codeType | attribute_list.xsd |
| layout-keyType | time_layout.xsd |
| measurement-categoryType | attribute_list.xsd |
| measurement-typeType | attribute_list.xsd |
| non-emptyString | attribute_list.xsd |
| offsetType | attribute_list.xsd |
| operational-modeType | head.xsd |
| origintypeType | origin.xsd |
| pavementTypeType | attribute_list.xsd |
| phone-numberType | attribute_list.xsd |
| physical-statusTypeType | attribute_list.xsd |
| precipitationTypeType | attribute_list.xsd |
| pressureTypeType | attribute_list.xsd |
| provinceType | location.xsd |
| qc-flagtypeType | qc.xsd |
| qc-flagvalueType | qc.xsd |
| radiationTypeType | attribute_list.xsd |
| radius-unitsType | location.xsd |
| scribe-accum-typeType | scribe-attribute_list.xsd |
| scribe-accum-unitsType | scribe-attribute_list.xsd |
| scibe-basic-simple-cloud-coverType | scribe-attribute_list.xsd |
| scribe-cloud-cover-ceiling-codeType | scribe-attribute_list.xsd |
| scribe-cloud-unitsType | scribe-attribute_list.xsd |
| scribe-excep-codeType | scribe-attribute_list.xsd |
| scribe-excep-typeType | scribe-attribute_list.xsd |
| scribe-freezing-spray-frequencyType | scribe-attribute_list.xsd |
| scribe-freezing-spray-ice-cover-modifierType | scribe-attribute_list.xsd |
| scribe-freezing-spray-intensityType | scribe-attribute_list.xsd |
| scribe-freezing-spray-occurType | scribe-attribute_list.xsd |
| scribe-langType | scribe-attribute_list.xsd |
| scribe-msc-zone-code-statusType | scribe-attribute_list.xsd |
| scribe-precipitation-frequencyType | scribe-attribute_list.xsd |
| scribe-precipitation-intensityType | scribe-attribute_list.xsd |
| scribe-precipitation-occurType | scribe-attribute_list.xsd |
| scribe-precipitation-typeType | scribe-attribute_list.xsd |
| scribe-simple-probabilityType | scribe-attribute_list.xsd |
| scribe-snow-level-unitsType | scribe-attribute_list.xsd |
| scribe-statusType | head.xsd |
| scribe-temperature-ground-frostType | scribe-attribute_list.xsd |
| scribe-temperature-trendType | scribe-attribute_list.xsd |
| scribe-temperature-typeType | scribe-attribute_list.xsd |
| scribe-temperature-unitsType | scribe-attribute_list.xsd |
| scribe-typeType | head.xsd |
| scribe-value-unitsType | scribe-attribute_list.xsd |
| scribe-visibility-frequencyType | scribe-attribute_list.xsd |
| scribe-visibility-typeType | scribe-attribute_list.xsd |
| scribe-warning-codeType | scribe-attribute_list.xsd |
| scribe-warning-statusType | scribe-attribute_list.xsd |
| scribe-warning-typeType | scribe-attribute_list.xsd |
| scribe-wave-height-ice-cover-modifierType | scribe-attribute_list.xsd |
| scribe-wave-height-unitsType | scribe-attribute_list.xsd |
| scribe-wind-directionType | scribe-attribute_list.xsd |
| scribe-wind-modificatorsType | scribe-attribute_list.xsd |
| scribe-wind-unitsType | scribe-attribute_list.xsd |
| slopeType | instrument.xsd |
| snapshot-cameraTypeType | attribute_list.xsd |
| snowTypeType | attribute_list.xsd |
| stateType | attribute_list.xsd |
| station-categoryType | location-metadata.xsd |
| stationStateType | location-metadata.xsd |
| station-typeType | location-metadata.xsd |
| subsurfaceTypeType | attribute_list.xsd |
| summarizationType | location.xsd |
| summaryType | qc.xsd |
| surfaceType | instrument.xsd |
| temperatureTypeType | attribute_list.xsd |
| time-coordinateType | time_layout.xsd |
| time-layoutAttributeType | parameters.xsd |
| timeUnitsType | location-metadata.xsd |
| timezonetypeType | attribute_list.xsd |
| unitType | attribute_list.xsd |
| US-state-codeType | attribute_list.xsd |
| visibilityTypeType | attribute_list.xsd |
| weatherTypeType | attribute_list.xsd |
| windTypeType | attribute_list.xsd |

- References

- Meteorological Development Laboratory National Digital Forecast Data XML (EXPERIMENTAL - Production) http://www.nws.noaa.gov/mdl/XML/production_xml.htm

- National Transportation Communications for ITS Protocol Object Definitions for (NTCIP)Environmental Sensor Stations (NTCIP 1204 v2.17.doc) http://www.ntcip.org/

- World Meteorological Organization (WMO)
http://www.wmo.ch/index-en.html

- W3C Technical Reports and Publications for XML Schema definition language http://www.w3.org/TR/xmlschema-0/

Glossary

CMML 	Canadian Meteorological Markup Language

DMF 		Data Management Framework

DWML		Digital Weather Markup Language

EC  		Environment Canada

ESS 		Environmental Sensor Station

IP		Internet Protocol

MTQ		Ministry of Transports Quebec

NOAA		National Oceanic and Atmospheric Administration

NTCIP 	National Transportation Communications for ITS Protocol

NWS		National Weather Service

PTC 		Provinces and Territories of Canada

QC 		Quality Control

RPU 		Remote Processing Unit

RWIN 		Road Weather Information Network

RWIS 		Road Weather Information System

RWISC 	Road Weather Information System for Canada

RWML		Road Web Markup Language

UTC		Universal Coordinated Time

WMO 		World Meteorological Organization

XML 		Extensible Markup Language

# Quick Reference

A

accum-amount	8

end	8

lower-limit	See lower-limit(accum)

start	8

type	8

upper-limit	See upper-limit(accum)

accum-list	7

accum-amount	8

units	8

air-quality(Obs)	39

index	39

qc	39

qualifier	39

type	39

value	39

air-quality(PF)	29

categorical-code	29

categorical-table	30

conversion-table	30

height	29

index	29

name	30

sampling-table	30

time-layout	30

type	29

units	29

value	30

air-quality-instrument	78

accuracy	78

calibration-history	79

cmml-index	78

commission-date	79

description	78

equipment-information	79

installation-date	79

instrument-id	79

instrument-state	78

last-inspection-date	79

last-maintenance-date	79

lower-range	78

point	79

resolution	78

sampling-interval	78

type	78

units	78

upper-range	79

area	19, 35

circle	19, 35

polygon	19, 36

rectangle	19, 36

B

bridge	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

aspect	48, 52, 56, 60, 64, 68, 72, 76, 81, 85, 89, 93, 97, 101

material	48, 52, 56, 60, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

pavement-surface-elevation	48, 52, 56, 60, 64, 68, 72, 77, 81, 85, 89, 93, 97, 101

slope	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 97, 101

surface	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 97, 101

water-surface-elevation	48, 52, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

C

calibrated-by	47, 51, 55, 59, 63, 67, 71, 75, 79, 83, 87, 91, 96, 100

calibration	47, 51, 55, 59, 63, 67, 71, 75, 79, 83, 87, 91, 95, 99

calibrated-by	47, 51, 55, 59, 63, 67, 71, 75, 79, 83, 87, 91, 96, 100

comment	47, 51, 55, 59, 63, 67, 71, 75, 80, 84, 88, 92, 96, 100

date	47, 51, 55, 59, 63, 67, 71, 75, 79, 83, 87, 91, 96, 100

calibration-history	47, 51, 55, 59, 63, 67, 71, 75, 79, 83, 87, 91, 95, 99

calibration	47, 51, 55, 59, 63, 67, 71, 75, 79, 83, 87, 91, 95, 99

categorical-definitions	102

categorical-table	102

categorical-table	102

categorical-key	103

category	102

units	102

value	103

circle	19, 35

point	19, 35

radius	19, 35

city	19

province	19

summarization	19

cloud-cover	8

ceiling-code	9

end	8

start	8

cloud-cover(excep)	9

ceiling-code	9

cover-end	9

cover-start	9

end	9

start	9

cloud-list	8

cloud-cover	8

exception	See exception(cloud)

units	8

cmml	3

data	3

head	3

version	3

contact	106

address	106

email	106

name	106

phone-number	106

conversion-definitions	103

conversion-table	103

conversion-table	103

conversion-key	103

end-value	103

equivalent-value	103

start-value	103

units	103

creation-date	4

refresh-frequency	4

D

data	3

forecast	7

meatdata	41

observation-series	35

E

elevation	105

datum	105

units	105

equipment-information	107

manufacturer	107

model-number	107

serial-number	107

excep-category	107

excep-code	107

excep-description	107

excep-type	107

excep-description	107

lang	107

exception(cloud)	9

cloud-cover	See cloud-cover(excep)

end	9

excep-category	9

start	9

exception(freezing)	10

end	10

excep-category	10

freezing-spray	10

start	10

exception(precip)	11

end	11

excep-category	11

precipitation-event	12

start	11

exception(temperature)	14

end	14

excep-category	14

start	14

temperature-value	14

exception(visibility)	15

end	15

excep-category	15

start	15

visibility	See visibility(MF)

exception(wind)	18

end	18

excep-category	18

start	18

wind	See wind(MF)

extension(Obs)	41

index	41

qc	41

qualifier	41

value	41

extension(PF)	34

categorical-code	34

categorical-table	34

conversion-table	34

height	34

index	34

name	35

sampling-table	34

time-layout	34

type	34

units	34

value	35

extension-instrument	98

accuracy	99

calibration-history	99

cmml-index	98

commission-date	99

description	98

equipment-information	99

installation-date	99

instrument-id	99

instrument-state	98

last-inspection-date	99

last-maintenance-date	99

lower-range	99

point	99

resolution	99

sampling-interval	99

sensor-name	98

units	98

upper-range	99

F

forecast	7

meteocode-forecast	7

pavement-forecast	18

freezing-spray	10

end	10

exception	See exception(freezing)

frequency	10

ice-cover-modifier	10

intensity	10

occurrence	10

start	10

freezing-spray-list	10

freezing-spray	10

G

gust-speed	18

lower-limit	18

upper-limit	18

H

head	3

product	4

source	5

humidity(Obs)	40

index	40

qc	40

qualifier	40

type	40

value	40

humidity(PF)	31

categorical-code	31

categorical-table	32

conversion-table	32

height	31

index	31

name	32

sampling-table	32

time-layout	31

type	31

units	31

value	32

humidity-instrument	82

accuracy	82

calibration-history	83

cmml-index	82

commission-date	83

description	82

equipment-information	83

installation-date	83

instrument-id	83

instrument-state	82

last-inspection-date	83

last-maintenance-date	83

lower-range	83

point	83

resolution	82

sampling-interval	83

type	82

units	82

upper-range	83

I

ice-cover	12

end	12

start	12

ice-cover-list	12

ice-cover	12

units	12

id	20, 35, 41

type	20, 35, 41

instrument

lane-number	47, 51, 55, 59, 63, 67, 71, 76, 80, 84, 88, 92, 96, 100

road-construction	47, 51, 55, 59, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

shadowing	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 98, 102

L

location	18

area	19

city	19

location-key	18

nws-zone	19

point	18

route	20

station	20

location(MD)	41

id	41

region-metadata	41

station-metadata	42

type	41

location(MF)	7

msc-zone-code	7

point	See point(MF)

location(MF)-zone-name	7

location(Obs)	35

area	35

point	35

route	36

lower-limit(accum)	8

cumul-inf	8

M

message	106

language	106

metadata	41

location	See location(MD)

supplementary-metadata	102

meteocode-forecast	7

location	See location(MF)

parameters	See parameters(MF)

msc-zone-code	7

status	7

msc-zone-name	7

lang	7

N

nws-zone	19

province	19

summarization	19

O

observation	36

air-quality	See air-quality(Obs)

extension	See extension(Obs)

humidity	See humidity(Obs)

pavement	See pavement(Obs)

physical-status	See physical-status(Obs)

precipitation	See precipitation(Obs)

pressure	See pressure(Obs)

radiation	See radiation(Obs)

snapshot-camera	See snapshot-camera(Obs)

snow	See snow(Obs)

subsurface	See subsurface(Obs)

temperature	See temperature(Obs)

valid-time	36

visibility	See visibility(Obs)

weather	See weather(Obs)

wind	See wind(Obs)

observation-series	35

location	See location(Obs)

observation	36

origin	35

origin	35

id	35

type	35

overpass	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

aspect	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

lower-roadway-elevation	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 98, 102

material	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

pavement-surface-elevation	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

slope	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

surface	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

P

parameters(MF)	7

accum-list	7

cloud-list	8

freezing-spray-list	10

ice-cover-list	12

precipitation-list	11

probability-of-precipitation-list	12

snow-level-list	13

temperature-list	13

UV-index-list	14

visibility-list	14

warning-list	16

wave-height-list	16

wind-list	17

parameters(PF)	21

air-quality	See air-quality(PF)

applicable-location	21

extension	See extension(PF)

humidity	See humidity(PF)

pavement	See pavement(PF)

physical-status	See physical-status(PF)

precipitation	See precipitation(PF)

pressure	See pressure(PF)

radiation	See radiation(PF)

snow	See snow(PF)

subsurface	See subsurface(PF)

temperature	See temperature(PF)

visibility	See visibility(PF)

weather	See weather(PF)

wind	See wind(PF)

pavement(Obs)	38

index	38

qc	38

qualifier	38

type	38

value	38

pavement(PF)	27

categorical-code	27

categorical-table	27

conversion-table	28

height	27

index	27

lane-number	27

name	28

sampling-table	28

time-layout	27

type	27

units	27

value	28

pavement-forecast	18

location	18

parameters	See parameters(PF)

time-layout	20

pavement-instrument	70

accuracy	70

calibration-history	71

cmml-index	70

commission-date	71

description	70

equipment-information	71

installation-date	71

instrument-id	71

instrument-state	70

last-inspection-date	71

last-maintenance-date	71

lower-range	70

point	71

resolution	70

sampling-interval	70

type	70

units	70

upper-range	70

physical-status(MD)	44

accuracy	45

description	45

instrument-state	45

lower-range	45

resolution	45

sampling-interval	45

type	44

units	45

upper-range	45

physical-status(Obs)	39

index	39

qc	39

qualifier	39

type	39

value	39

physical-status(PF)	30

categorical-code	30

categorical-table	31

conversion-table	31

name	31

sampling-table	31

time-layout	30

type	30

units	30

value	31

point	105

elevation	105

latitude	105

longitude	105

point(MF)	7

latitude	7

longitude	7

polygon	19

point	20, 36

polygon	36

precipitation(Obs)	37

index	37

qc	37

qualifier	37

type	37

value	37

precipitation(PF)	24

categorical-code	24

categorical-table	24

conversion-table	24

height	24

index	24

name	25

sampling-table	25

time-layout	24

type	24

units	24

value	25

precipitation-event(L1)	11

end	11

exception	See exception(precip)

frequency	11

intensity	11

occurrence	11

precipitation-event	See precipitation-event(L1)

start	11

type	11

precipitation-instrument	58

accuracy	58

calibration-history	59

cmml-index	58

commission-date	59

description	58

equipment-information	59

installation-date	59

instrument-id	58

instrument-state	58

last-inspection-date	59

last-maintenance-date	59

lower-range	58

point	59

resolution	58

sampling-interval	58

type	58

units	58

upper-range	58

precipitation-list	11

precipitation-event	See precipitation-event(L1)

pressure(Obs)	36

index	36

qc	36

qualifier	36

type	36

value	36

pressure(PF)	21

categorical-code	21

categorical-table	21

conversion-table	22

height	21

index	21

name	22

sampling-table	22

time-layout	21

type	21

units	21

value	22

pressure-instrument	45

accuracy	46

calibration-history	47

cmml-index	46

commission-date	47

description	46

equipment-information	46

installation-date	47

instrument-id	46

instrument-state	46

last-inspection-date	47

last-maintenance-date	47

lower-range	46

point	46

resolution	46

sampling-interval	46

type	46

units	46

upper-range	46

probability-of-precipitation	12

end	13

start	12

probability-of-precipitation-list	12

probability-of-precipitation	12

unit	12

product	4

amended-regions	5

category	4

creation-date	4

current-issue	5

field	4

initials	5

next-issue	5

operational-mode	4

status	5

timezone	5

title	4

type	5

valid-begin-time	5

valide-end-time	5

production-center	5

sub-center	5

Q

qc	106

performer	106

qc-flag	106

summary	106

qc-flag	106

associated-measurement-type	106

associated-meaurement-category	106

message	106

type	106

value	106

qualifier	105

type	105

units	105

R

radiation(Obs)	38

index	38

qc	38

qualifier	38

type	38

value	38

radiation(PF)	25

categorical-code	25

categorical-table	25

conversion-table	25

height	25

index	25

name	26

sampling-table	26

time-layout	25

type	25

units	25

value	26

radiation-instrument	62

accuracy	62

calibration-history	63

cmml-index	62

commission-date	63

description	62

equipment-information	63

installation-date	63

instrument-id	63

instrument-state	62

last-inspection-date	63

last-maintenance-date	63

lower-range	62

point	63

resolution	62

sampling-interval	62

type	62

units	62

upper-range	62

radius	19, 35

radius-units	19, 35

rectangle	19, 36

point	19, 36

region-metadata	41

area	42

city	42

climatic-zone	42

country	42

data-owner	42

description	42

district	42

name	41

province	42

route	42

state	42

timezone	42

road	47, 51, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

aspect	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

earthwork	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

pavement-surface-elevation	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

slope	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

surface	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

road-construction	47, 51, 55, 59, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

bridge	48, 52, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

overpass	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 97, 101

road	47, 51, 56, 60, 64, 68, 72, 76, 80, 84, 88, 92, 96, 100

route	20, 36

point	20, 36

rpu	44

cmml-index	44

equipment-information	44

ip-address	44

phone-number	44

physical-status	See physical-status(MD)

S

sampling-definitions	103

sampling-table	103

sampling-interval	103

units	104

sampling-table	103

number-of-samples	104

sampling-interval	103

sampling-key	103

sampling-start	104

shadowing	49, 53, 57, 61, 65, 69, 73, 77, 81, 85, 89, 93, 98, 102

end-date	49, 53, 57, 61, 65, 69, 73, 78, 82, 86, 90, 94, 98, 102

shadowing-ends	49, 53, 58, 62, 66, 70, 74, 78, 82, 86, 90, 94, 98, 102

shadowing-starts	49, 53, 57, 62, 66, 70, 74, 78, 82, 86, 90, 94, 98, 102

start-date	49, 53, 57, 61, 65, 69, 73, 77, 82, 86, 90, 94, 98, 102

snapshot-camera(Obs)	40

index	41

qc	41

qualifier	41

type	41

value	41

snapshot-camera-instrument	94

accuracy	94

calibration-history	95

cmml-index	94

commission-date	95

description	94

equipment-information	95

installation-date	95

instrument-id	95

instrument-state	94

last-inspection-date	95

last-maintenance-date	95

lower-range	95

point	95

resolution	94

sampling-interval	95

type	94

units	94

upper-range	95

snow(Obs)	40

index	40

qc	40

qualifier	40

type	40

value	40

snow(PF)	32

categorical-code	32

categorical-table	32

conversion-table	33

height	32

index	32

name	33

sampling-table	33

time-layout	32

type	32

units	32

value	33

snow-instrument	86

accuracy	86

calibration-history	87

cmml-index	86

commission-date	87

description	86

equipment-information	87

installation-date	87

instrument-id	87

instrument-state	86

last-inspection-date	87

last-maintenance-date	87

lower-range	87

point	87

resolution	86

sampling-interval	86

type	86

units	86

upper-range	87

snow-level	13

end	13

start	13

snow-level-list	13

snow-level	13

unit	13

source	5

credit	6

credit-logo	6

disclaimer	5

feedback	6

more-information	5

production-center	5

start-valid-time	21

period-name	21

station	20

id	20

type	20

station-metadata	42

air-quality-instrument	78

category	43

city	43

climatic-zone	43

commission-date	44

country	43

data-owner	43

description	43

district	43

extension-instrument	98

humidity-instrument	82

installation-date	44

last-inspection-date	44

last-maintenance-date	44

name	42

pavement-instrument	70

point	43

precipitation-instrument	58

pressure-instrument	45

province	43

radiation-instrument	62

reporting-frequency	45

rpu	44

snapshot-camera-instrument	94

snow-instrument	86

state	43

station-maintainer	44

station-owner	44

station-state	43

subsurface-instrument	74

temperature-instrument	54

timezone	43

type	44

vegetation	45

visibility-instrument	66

weather-instrument	90

wind-instrument	50

subsurface(Obs)	39

index	39

qc	39

qualifier	39

type	39

value	39

subsurface(PF)	28

categorical-code	28

categorical-table	29

conversion-table	29

index	28

lane-number	28

name	29

sampling-table	29

time-layout	29

type	28

units	28

value	29

subsurface-instrument	74

accuracy	74

calibration-history	75

cmml-index	74

commission-date	75

description	74

equipment-information	75

installation-date	75

instrument-id	75

instrument-state	74

last-inspection-date	75

last-maintenance-date	75

lower-range	74

point	75

resolution	74

sampling-interval	74

type	74

units	74

upper-range	74

summarization	105

supplementary-metadata	102

categorical-definitions	102

conversion-definitions	103

data-owner	102

sampling-definitions	103

T

temperature(Obs)	37

index	37

qc	37

qualifier	37

type	37

value	37

temperature(PF)	23

categorical-code	23

categorical-table	23

conversion-table	23

height	23

index	23

name	24

sampling-table	24

time-layout	23

type	23

units	23

value	24

temperature-instrument	54

accuracy	54

calibration-history	55

cmml-index	54

commission-date	55

description	54

equipment-information	55

installation-date	55

instrument-id	54

instrument-state	54

last-inspection-date	55

last-maintenance-date	55

lower-range	54

point	54

resolution	54

sampling-interval	54

type	54

units	54

upper-range	54

temperature-list	13

exception	See exception(temperature)

temperature-value	13

type	13

units	13

temperature-value	13

end	13

ground-frost	13

limit	14

lower-limit	14

start	13

trend	13

upper-limit	14

time-layout	20, 105

end-valid-time	21

layout-key	20

start-valid-time	21

summarization	20

time-coordinate	20

U

upper-limit(accum)	8

cumul-sup	8

UV-index	14

end	14

start	14

UV-index-list	14

UV-index	14

V

value	105

units	106

value(MF)	15

units	15

vegetation	45

age	45

direction	45

height	45

proximity	45

type	45

visibility(MF)	15

end	15

exception	See exception(visibility)

frequency	15

start	15

type	15

value	See value(MF)

visibility(Obs)	38

index	38

qc	38

qualifier	38

type	38

value	38

visibility(PF)	26

categorical-code	26

categorical-table	26

conversion-table	26

height	26

index	26

name	27

sampling-table	26

time-layout	26

type	26

units	26

value	27

visibility-instrument	66

accuracy	66

calibration-history	67

cmml-index	66

commission-date	67

description	66

equipment-information	67

installation-date	67

instrument-id	67

instrument-state	66

last-inspection-date	67

last-maintenance-date	67

lower-range	66

point	67

resolution	66

sampling-interval	66

type	66

units	66

upper-range	66

visibility-list	14

visibility	See visibility(MF)

W

waring-list	16

warning-event	See warning-event(L1)

warning-event(L1)	16

code	16

end	16

start	16

status	16

type	16

warning-event	See warning-event(L1)

wave-height	16

end	17

ice-cover-modifier	17

lower-limit	17

start	16

upper-limit	17

wave-height-list	16

units	16

wave-height	16

weather(Obs)	40

index	40

qc	40

qualifier	40

type	40

value	40

weather(PF)	33

categorical-code	33

categorical-table	33

conversion-table	33

height	33

index	33

name	34

sampling-table	34

time-layout	33

type	33

units	33

value	34

weather-instrument	90

accuracy	90

calibration-history	91

cmml-index	90

commission-date	91

description	90

equipment-information	91

installation-date	91

instrument-id	91

instrument-state	90

last-inspection-date	91

last-maintenance-date	91

lower-range	91

point	91

resolution	90

sampling-interval	91

type	90

units	90

upper-range	91

wind(MF)	17

direction	17

end	17

exception	See exception(wind)

gust-speed	18

modificatior	17

start	17

wind-speed	17

wind(Obs)	37

index	37

qc	37

qualifier	37

type	37

value	37

wind(PF)	22

categorical-code	22

categorical-table	22

conversion-table	22

height	22

index	22

name	23

sampling-table	23

time-layout	22

type	22

units	22

value	23

wind-instrument	50

accuracy	50

calibration-history	51

cmml-index	50

commission-date	51

description	50

equipment-information	51

installation-date	51

instrument-id	50

instrument-state	50

last-inspection-date	51

last-maintenance-date	51

lower-range	50

point	50

resolution	50

sampling-interval	50

type	50

units	50

upper-range	50

wind-list	17

units	17

wind	See wind(MF)

wind-speed	17

lower-limit	17

upper-limit	18

It is required when CMML document contains <meteocode-forecast> element.

It is required when CMML document contains <meteocode-forecast> element.

2 Pavement index is also associated to a puck. For instance, index 1 represents the pavement value from puck 1

3 Subsurface index is also associated to a puck. For instance, index 1 represents the subsurface value from puck 1.

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive

The French values are not available in this CMML schema version

When the Description in the tables is not filled it is either not applicable or the term is auto descriptive